/*

* Copyright (c) 2016, Freescale Semiconductor, Inc.

* Copyright 2016-2019 NXP

* All rights reserved.

*

* FreeRTOS移植版本 - 最终完整版（新增SPI→UDP转发）

* 功能：1. CAN2→SPI联动 2. SPI→CAN1 CAN FD转发(64字节) 3. SPI→UDP转发(64字节) 4. 完整软件看门狗(监控所有任务)

* 看门狗：监控9个任务，超时5秒打印日志并重置，优先级最高

*/

#include "Cpu.h"

#include <stdio.h>

#include <string.h>

#include <stdint.h>

#include <stdbool.h>

#include <stdlib.h>

#include <stdarg.h>

#include "clockMan1.h"

#include "canCom1.h"     // CAN FD模式

#include "canCom2.h"     // 标准CAN模式

#include "dspi1.h"       // SPI

#include "dmaController1.h"

#include "pin_mux.h"

#include "uart_pal1.h"

#if CPU_INIT_CONFIG

#include "Init_Config.h"

#endif

/* FreeRTOS头文件 */

#include "FreeRTOS.h"

#include "task.h"

#include "queue.h"

#include "semphr.h"

#include "event_groups.h"

/* ============== 新增：以太网UDP相关头文件 ============== */

#include "lwip/opt.h"

#include "lwip/sys.h"

#include "lwip/timeouts.h"

#include "lwip/debug.h"

#include "lwip/stats.h"

#include "lwip/init.h"

#include "lwip/tcpip.h"

#include "lwip/netif.h"

#include "lwip/api.h"

#include "lwip/arch.h"

#include "lwip/udp.h"

#include "netif/ethernet.h"

#include "lwipcfg.h"

#include "enetif.h"

/* ============== 以太网UDP头文件结束 ============== */

#define LOG(...) uart_log(__VA_ARGS__)

/* 系统复位函数 */

void NVIC_SystemReset(void)

{

const uint32_t AIRCR_VECTKEY = (0x5FAUL << 16);

volatile uint32_t *AIRCR = (volatile uint32_t *)0xE000ED0CUL;

uint32_t reg = *AIRCR;

reg = (reg & 0x700U) | AIRCR_VECTKEY | (1UL << 2);

*AIRCR = reg;

for (;;) { ; }

}

/******************************************************************************

* 硬件配置定义（含CAN FD+看门狗+UDP宏）

******************************************************************************/

#define LED_PORT        PTA

#define LED0            10U

#define LED1            7U

#define LED2            4U

/* UART配置 */

#define TIMEOUT         1000UL

#define BUFFER_SIZE     128UL

#define UART_SEND_BUF   64UL

/* CAN配置（CAN FD核心宏） */

#define CAN_LED_CTRL_ID_LED0_ON  0x123UL

#define CAN_LED_CTRL_ID_LED1_ON  0x456UL

#define CAN_LED_CTRL_DATA_TOGGLE 0x01U

#define CAN_LED_CTRL_DATA_OFF    0x02U

#define CAN_LED_CTRL_DATA_ON     0x03U

#define CANFD_MAX_DLEN      64U        // CAN FD最大数据长度

#define CANFD_TX_FORWARD_ID 0x300UL    // SPI转发CAN FD的固定ID

#define CAN1_TX_MAILBOX     1UL        // CAN1发送邮箱

/* SPI配置（64字节收发） */

#define SPI_TRANSFER_TIMEOUT     1000U

#define SPI_FRAME_SIZE           64U

#define SPI_CAN_PACKAGE_SIZE     13U

#define SPI_TRANSFER_INTERVAL_MS 10U

/* 软件看门狗配置（新增UDP相关监控项） */

#define WATCHDOG_TIMEOUT_MS    5000U    // 任务超时时间：5秒

#define WATCHDOG_CHECK_MS      1000U    // 看门狗检查周期：1秒

/* ============== 新增：UDP配置 ============== */

#define UDP_SEND_TASK_STACK_SIZE  1024   // UDP任务栈大小

#define UDP_SEND_TASK_PRIORITY    (tskIDLE_PRIORITY + 2) // UDP任务优先级

#define UDP_TARGET_PORT           8089   // 目标UDP端口

#define UDP_LOCAL_PORT            44444  // 本地UDP端口

#define UDP_TARGET_IP             "192.168.5.2" // 目标IP地址（可根据实际修改）

/* ============== UDP配置结束 ============== */

/* 看门狗任务枚举（新增UDP相关项） */

typedef enum {

WD_CAN_INIT = 0,

WD_SPI_INIT,

WD_UART,

WD_LED,

WD_CAN2_RX,

WD_SPI_TX,

WD_SPI_RX,

WD_CAN1_TX,

WD_UDP_SEND,        // 新增：监控UDP发送任务

WD_ETH_INIT,        // 新增：监控以太网初始化任务

WD_COUNT            // 总任务数：10个

} WatchdogId_t;

static volatile TickType_t g_watchdog_last_tick[WD_COUNT];

/******************************************************************************

* 数据结构体定义（新增SPI→UDP转发结构体）

******************************************************************************/

/* SPI接收数据结构体（含时间戳） */

typedef struct {

uint32_t msgId;

uint8_t data[SPI_FRAME_SIZE];

uint8_t dataLen;

uint32_t sendTimestamp;

uint32_t recvTimestamp;

} SpiRxData;

/* ============== 新增：SPI→UDP转发数据结构体 ============== */

typedef struct {

uint32_t spi_raw_id;    // SPI接收的原始ID

uint8_t  spi_data[64];  // SPI接收的64字节原始数据

uint32_t roundTripTime; // SPI往返时间（可选，用于调试）

} SpiRawData_UDP;

/* ============== UDP数据结构体结束 ============== */

/******************************************************************************

* FreeRTOS对象定义（含CAN1+看门狗+UDP）

******************************************************************************/

QueueHandle_t xCanRxQueue;        /* CAN2接收队列 */

QueueHandle_t xSpiTxQueue;        /* SPI发送队列 */

QueueHandle_t xSpiRxQueue;        /* SPI接收队列 */

QueueHandle_t xCan1TxQueue;       /* CAN1 CAN FD发送队列 */

QueueHandle_t xUdpSendQueue;      /* 新增：UDP发送队列（存储SPI原始数据） */

SemaphoreHandle_t xUartMutex;     /* UART互斥锁 */

SemaphoreHandle_t xCanMutex;      /* CAN2互斥锁 */

SemaphoreHandle_t xSpiMutex;      /* SPI互斥锁 */

SemaphoreHandle_t xCan1Mutex;     /* CAN1互斥锁 */

TaskHandle_t xCanInitTaskHandle;

TaskHandle_t xSpiInitTaskHandle;

TaskHandle_t xUartTaskHandle;

TaskHandle_t xLedTaskHandle;

TaskHandle_t xCan2RxTaskHandle;   /* CAN2接收任务 */

TaskHandle_t xSpiTxTaskHandle;    /* SPI发送任务 */

TaskHandle_t xSpiRxTaskHandle;    /* SPI接收任务 */

TaskHandle_t xCan1TxTaskHandle;   /* CAN1 CAN FD发送任务 */

TaskHandle_t xUdpSendTaskHandle;  /* 新增：UDP发送任务句柄 */

TaskHandle_t xEthInitTaskHandle;  /* 新增：以太网初始化任务句柄 */

EventGroupHandle_t xSystemEvents;

/* 事件标志位定义（新增以太网相关） */

#define EVENT_BUTTON1_PRESSED    (1 << 1)

#define EVENT_UART_TX_READY      (1 << 2)

#define EVENT_INIT_COMPLETE      (1 << 3)

#define EVENT_CAN_DATA_RECEIVED  (1 << 4)

#define EVENT_SPI_RX_DATA        (1 << 5)

#define EVENT_FUNC_STARTED       (1 << 6)

#define EVENT_SPI2CAN_READY      (1 << 7)

#define EVENT_ETH_INIT_COMPLETE  (1 << 8) // 新增：以太网初始化完成事件

/******************************************************************************

* 全局变量（新增以太网相关）

******************************************************************************/

uint8_t uartRxBuffer[BUFFER_SIZE];

uint8_t bufferIdx = 0;

volatile bool rxComplete = false;

uint8_t spi_tx_buffer[SPI_FRAME_SIZE];  /* SPI发送缓冲区 */

uint8_t spi_rx_buffer[SPI_FRAME_SIZE];  /* SPI接收缓冲区 */

/* 外部声明DSPI1状态变量 */

extern dspi_state_t dspi1State;

/* ============== 新增：以太网全局变量 ============== */

struct netif netif;                // 网络接口结构体

#if LWIP_DHCP

struct dhcp netif_dhcp;            // DHCP结构体（如需动态IP启用）

#endif

#if LWIP_AUTOIP

struct autoip netif_autoip;        // AUTOIP结构体（如需自动IP启用）

#endif

/* ============== 以太网全局变量结束 ============== */

/******************************************************************************

* 函数声明（含看门狗+CAN1+SPI联动+UDP）

******************************************************************************/

void RTOS_Objects_Init_Base(void);

void RTOS_Objects_Init_Func(void);

void CAN_Init_Task(void *pvParameters);

void SPI_Init_Task(void *pvParameters);

void UART_Task(void *pvParameters);

void LED_Task(void *pvParameters);

void CAN2_Rx_Task(void *pvParameters);

void SPI_Tx_Task(void *pvParameters);

void SPI_Rx_Task(void *pvParameters);

void CAN1_Tx_Task(void *pvParameters);

void uartRxCallback(void *driverState, uart_event_t event, void *userData);

void GPIO_Init(void);

void Board_Init(void);

void uart_log(const char *fmt, ...);

void CANCOM1_Init(void);

void CANCOM2_Init(void);

void DSPI_Init(void);

void UART_PrintCANMsg(uint32_t msgId, uint8_t *data, uint8_t len);

void LED_ControlByCAN(uint32_t msgId, uint8_t *data, uint8_t dataLen);

void SPI_SendCANMessage(uint32_t msgId, uint8_t *data, uint8_t dataLen);

void ProcessAndForwardSPI2CANData(SpiRxData *spiRxData);

/* 看门狗函数 */

void Watchdog_Pet(WatchdogId_t id);

static void Watchdog_Task(void *pvParameters);

/* ============== 新增：UDP相关函数声明 ============== */

void Ethernet_Init_Task(void *pvParameters);  // 以太网初始化任务

static void udp_send_task(void *pvParameters); // UDP发送任务

static void enetif_init(void);                // 网络接口初始化

static void eth_test_init(void* arg);         // 以太网测试初始化

/* ============== UDP函数声明结束 ============== */

/******************************************************************************

* UART接收回调函数（保留）

******************************************************************************/

void uartRxCallback(void *driverState, uart_event_t event, void *userData)

{

BaseType_t xHigherPriorityTaskWoken = pdFALSE;

(void)driverState;

(void)userData;

if (event == UART_EVENT_RX_FULL)

{

if (uartRxBuffer[bufferIdx] == '\n' ||

uartRxBuffer[bufferIdx] == '\r' ||

bufferIdx == (BUFFER_SIZE - 1))

{

rxComplete = true;

xEventGroupSetBitsFromISR(xSystemEvents, EVENT_UART_TX_READY,

&xHigherPriorityTaskWoken);

}

else

{

bufferIdx++;

UART_ReceiveData(&uart_pal1_instance, &uartRxBuffer[bufferIdx], 1U);

}

}

portYIELD_FROM_ISR(xHigherPriorityTaskWoken);

}

/******************************************************************************

* CAN初始化任务（含看门狗喂狗）

******************************************************************************/

void CAN_Init_Task(void *pvParameters)

{

(void)pvParameters;

LOG("开始CAN初始化...\r\n");

/* 1. 初始化CANCOM2（标准CAN模式） */

LOG("正在初始化CANCOM2（标准CAN模式）...\r\n");

status_t status_can2 = FLEXCAN_DRV_Init(INST_CANCOM2, &canCom2_State, &canCom2_InitConfig0);

if (status_can2 == STATUS_SUCCESS) {

LOG("CANCOM2 初始化成功\r\n");

} else {

LOG("CANCOM2 初始化失败 (状态: %d)\r\n", status_can2);

}

vTaskDelay(pdMS_TO_TICKS(100));

/* 2. 初始化CANCOM1（CAN FD模式） */

LOG("正在初始化CANCOM1（CAN FD模式）...\r\n");

status_t status_can1 = FLEXCAN_DRV_Init(INST_CANCOM1, &canCom1_State, &canCom1_InitConfig0);

if (status_can1 == STATUS_SUCCESS) {

LOG("CANCOM1 初始化成功\r\n");

} else {

LOG("CANCOM1 初始化失败 (状态: %d)\r\n", status_can1);

}

LOG("================================================\r\n");

LOG("CAN初始化完成: CANCOM2(%s), CANCOM1(%s)\r\n",

status_can2 == STATUS_SUCCESS ? "成功" : "失败",

status_can1 == STATUS_SUCCESS ? "成功" : "失败");

LOG("即将启动所有CAN-SPI-UDP联动功能...\r\n");

LOG("================================================\r\n");

xEventGroupSetBits(xSystemEvents, EVENT_INIT_COMPLETE);

RTOS_Objects_Init_Func();

/* 任务循环：喂看门狗（WD_CAN_INIT） */

for(;;)

{

Watchdog_Pet(WD_CAN_INIT);

vTaskDelay(pdMS_TO_TICKS(2000));

}

}

/******************************************************************************

* SPI初始化任务（含看门狗喂狗）

******************************************************************************/

void SPI_Init_Task(void *pvParameters)

{

(void)pvParameters;

LOG("开始SPI初始化...\r\n");

DSPI_Init();

/* 任务循环：喂看门狗（WD_SPI_INIT） */

for(;;)

{

Watchdog_Pet(WD_SPI_INIT);

vTaskDelay(pdMS_TO_TICKS(2000));

}

}

/******************************************************************************

* CAN2接收任务（含看门狗喂狗）

******************************************************************************/

void CAN2_Rx_Task(void *pvParameters)

{

flexcan_msgbuff_t recvMsg;

(void)pvParameters;

xEventGroupWaitBits(xSystemEvents, EVENT_FUNC_STARTED, pdFALSE, pdFALSE, portMAX_DELAY);

flexcan_data_info_t std_rx_info = {

.data_length = 8U,

.msg_id_type = FLEXCAN_MSG_ID_STD,

.enable_brs = false,

.fd_enable = false,

.fd_padding = 0U

};

FLEXCAN_DRV_ConfigRxMb(INST_CANCOM2, 0, &std_rx_info, 0);

FLEXCAN_DRV_SetRxMbGlobalMask(INST_CANCOM2, FLEXCAN_MSG_ID_STD, 0U);

LOG("CAN2接收任务启动成功 - 已准备接收CAN消息\r\n");

/* 任务循环：喂看门狗（WD_CAN2_RX） */

for(;;)

{

if (FLEXCAN_DRV_Receive(INST_CANCOM2, 0, &recvMsg) == STATUS_SUCCESS && recvMsg.dataLen > 0)

{

UART_PrintCANMsg(recvMsg.msgId, recvMsg.data, recvMsg.dataLen);

LED_ControlByCAN(recvMsg.msgId, recvMsg.data, recvMsg.dataLen);

SPI_SendCANMessage(recvMsg.msgId, recvMsg.data, recvMsg.dataLen);

if(xQueueSend(xCanRxQueue, &recvMsg, portMAX_DELAY) == pdPASS)

{

xEventGroupSetBits(xSystemEvents, EVENT_CAN_DATA_RECEIVED);

}

memset(&recvMsg, 0, sizeof(recvMsg));

}

Watchdog_Pet(WD_CAN2_RX);

vTaskDelay(pdMS_TO_TICKS(2));

}

}

/******************************************************************************

* SPI发送任务（含看门狗喂狗）

******************************************************************************/

/******************************************************************************

* SPI发送任务（最终微调版：手动构造 FF 00 开头数据 + 解决 SPI 滞后）

******************************************************************************/

void SPI_Tx_Task(void *pvParameters)

{

flexcan_msgbuff_t spiMsg;

(void)pvParameters;

// 1. 等待系统功能启动事件

xEventGroupWaitBits(xSystemEvents, EVENT_FUNC_STARTED, pdFALSE, pdFALSE, portMAX_DELAY);

LOG("SPI发送任务启动成功 - 构造数据格式: FF 00 ID_H ID_L LEN...\r\n");

// 给系统一点稳定时间

vTaskDelay(pdMS_TO_TICKS(100));

for(;;)

{

// 2. 从队列获取由 CAN2 接收到的消息

if(xQueueReceive(xSpiTxQueue, &spiMsg, pdMS_TO_TICKS(SPI_TRANSFER_INTERVAL_MS)) == pdTRUE)

{

// 3. 获取 SPI 互斥锁，防止多任务冲突

if(xSemaphoreTake(xSpiMutex, pdMS_TO_TICKS(50)) == pdTRUE)

{

uint32_t sendTime = (uint32_t)(xTaskGetTickCount() * portTICK_PERIOD_MS);

// --- ✅ 关键修改：手动构造你要求的特定 64 字节数据流 ---

memset(spi_tx_buffer, 0, SPI_FRAME_SIZE);

// 字节 0-1: 强制固定为 FF 00

spi_tx_buffer[0] = 0xFF;

spi_tx_buffer[1] = 0x00;

// 字节 2-3: 填充 CAN ID (大端序手动排列，确保总线顺序)

spi_tx_buffer[2] = (uint8_t)((spiMsg.msgId >> 8) & 0xFF); // ID 高位

spi_tx_buffer[3] = (uint8_t)(spiMsg.msgId & 0xFF);        // ID 低位

// 字节 4: 填充数据长度

spi_tx_buffer[4] = spiMsg.dataLen;

// 字节 5-12: 拷贝 8 字节 CAN 载荷（对应 FPGA 截取的明文区）

memcpy(&spi_tx_buffer[5], spiMsg.data, (spiMsg.dataLen > 8) ? 8 : spiMsg.dataLen);

// 其余字节已经在 memset 中清零

// ---------------------------------------------------

// 4. 第一次传输：将数据发送给 FPGA（触发 FPGA 加密）

status_t spi_status = DSPI_MasterTransferBlocking(

INST_DSPI1,

spi_tx_buffer,

spi_rx_buffer,

SPI_FRAME_SIZE,

SPI_TRANSFER_TIMEOUT

);

if (spi_status == STATUS_SUCCESS)

{

// 5. 等待 FPGA 计算完成（给 FPGA 留出加密计算时间）

vTaskDelay(pdMS_TO_TICKS(1));

// 6. 第二次传输：发送 Dummy 数据，把刚刚算好的结果读回来

uint8_t dummy_tx[SPI_FRAME_SIZE] = {0};

spi_status = DSPI_MasterTransferBlocking(

INST_DSPI1,

dummy_tx,

spi_rx_buffer,

SPI_FRAME_SIZE,

SPI_TRANSFER_TIMEOUT

);

if (spi_status == STATUS_SUCCESS)

{

LOG("SPI Transfer Success! 已获取 ID:0x%03lX 的密文\r\n", spiMsg.msgId);

// 封装接收到的密文数据

SpiRxData rxData;

rxData.msgId = spiMsg.msgId;

memcpy(rxData.data, spi_rx_buffer, SPI_FRAME_SIZE);

rxData.dataLen = SPI_FRAME_SIZE;

rxData.sendTimestamp = sendTime;

// 发送到接收处理任务（进行后续的 CAN FD 转发或 UDP 转发）

if (xQueueSend(xSpiRxQueue, &rxData, 0) == pdPASS)

{

xEventGroupSetBits(xSystemEvents, EVENT_SPI_RX_DATA);

}

// 翻转 LED 指示灯

PINS_DRV_TogglePins(LED_PORT, (1 << LED1));

}

}

if (spi_status != STATUS_SUCCESS)

{

LOG("SPI 传输失败! 错误码: %d\r\n", spi_status);

}

// 释放 SPI 互斥锁

xSemaphoreGive(xSpiMutex);

}

}

// 7. 喂看门狗，防止任务卡死导致系统重启

Watchdog_Pet(WD_SPI_TX);

// 任务小休，让出 CPU

vTaskDelay(pdMS_TO_TICKS(2));

}

}

/******************************************************************************

* SPI接收任务（修改：新增SPI→UDP转发逻辑）

******************************************************************************/

void SPI_Rx_Task(void *pvParameters)

{

SpiRxData rxData;

EventBits_t uxBits;

(void)pvParameters;

xEventGroupWaitBits(xSystemEvents, EVENT_FUNC_STARTED, pdFALSE, pdFALSE, portMAX_DELAY);

LOG("SPI接收任务启动成功 - 等待从机响应数据...\r\n");

/* 任务循环：喂看门狗（WD_SPI_RX） */

for(;;)

{

uxBits = xEventGroupWaitBits(

xSystemEvents,

EVENT_SPI_RX_DATA,

pdTRUE,

pdFALSE,

pdMS_TO_TICKS(1000)

);

if((uxBits & EVENT_SPI_RX_DATA) != 0)

{

while(xQueueReceive(xSpiRxQueue, &rxData, 0) == pdTRUE)

{

rxData.recvTimestamp = (uint32_t)(xTaskGetTickCount() * portTICK_PERIOD_MS);

uint32_t roundTripTime = rxData.recvTimestamp - rxData.sendTimestamp;

uint32_t canId = *(uint32_t *)rxData.data;

LOG("================================================\r\n");

LOG("SPI Round-trip time: %lu ms\r\n", roundTripTime);

LOG("CAN ID:0x%03lX | 64字节SPI完整接收数据:\r\n", canId);

for (uint8_t i = 0; i < SPI_FRAME_SIZE; i++)

{

LOG("%02X ", rxData.data[i]);

if((i+1) % 16 == 0) LOG("\r\n");

}

LOG("\r\n================================================\r\n\r\n");

PINS_DRV_TogglePins(LED_PORT, (1 << LED0));

// 1. SPI→CAN1转发（原有逻辑）

ProcessAndForwardSPI2CANData(&rxData);

xEventGroupSetBits(xSystemEvents, EVENT_SPI2CAN_READY);

// 2. 新增：SPI→UDP转发逻辑

SpiRawData_UDP udpData;

udpData.spi_raw_id = canId;

memcpy(udpData.spi_data, rxData.data, SPI_FRAME_SIZE);

udpData.roundTripTime = roundTripTime;

// 将数据加入UDP发送队列

if(xQueueSend(xUdpSendQueue, &udpData, pdMS_TO_TICKS(100)) != pdPASS)

{

LOG("警告: UDP发送队列已满，丢弃SPI数据 ID:0x%03lX\r\n", canId);

}

else

{

LOG("SPI→UDP: 数据 ID:0x%03lX 已加入UDP发送队列\r\n", canId);

}

}

}

else

{

static uint32_t timeout_count = 0;

timeout_count++;

if (timeout_count % 10 == 0)

{

LOG("SPI Rx: 等待从机响应... (超时计数: %lu)\r\n", timeout_count);

}

}

Watchdog_Pet(WD_SPI_RX);

vTaskDelay(pdMS_TO_TICKS(10));

}

}

/******************************************************************************

* SPI→CAN1数据处理函数（64字节直接转发，保留原有逻辑）

******************************************************************************/

void ProcessAndForwardSPI2CANData(SpiRxData *spiRxData)

{

flexcan_msgbuff_t forwardMsg;

// CAN FD消息配置：固定ID 0x300，长度64字节

forwardMsg.msgId = CANFD_TX_FORWARD_ID;

forwardMsg.dataLen = CANFD_MAX_DLEN;

// 直接复制SPI接收的64字节数据

memcpy(forwardMsg.data, spiRxData->data, SPI_FRAME_SIZE);

// 打印转发日志

LOG("SPI→CAN1转发: 固定ID=0x%03lX 数据长度=64字节\r\n", CANFD_TX_FORWARD_ID);

// 推入CAN1发送队列

if (xQueueSend(xCan1TxQueue, &forwardMsg, pdMS_TO_TICKS(100)) != pdPASS) {

LOG("警告: CAN1发送队列已满\r\n");

}

}

/******************************************************************************

* CAN1 CAN FD发送任务（含看门狗喂狗+重试机制，保留原有逻辑）

******************************************************************************/

void CAN1_Tx_Task(void *pvParameters)

{

flexcan_msgbuff_t txMsg;

(void)pvParameters;

xEventGroupWaitBits(xSystemEvents, EVENT_FUNC_STARTED, pdFALSE, pdFALSE, portMAX_DELAY);

LOG("CAN1 CAN FD发送任务启动成功 - 支持64字节发送\r\n");

/* 任务循环：喂看门狗（WD_CAN1_TX） */

for(;;)

{

if(xQueueReceive(xCan1TxQueue, &txMsg, pdMS_TO_TICKS(100)) == pdTRUE)

{

if(xSemaphoreTake(xCan1Mutex, pdMS_TO_TICKS(100)) == pdTRUE)

{

// CAN FD模式核心配置

flexcan_data_info_t fd_dataInfo = {

.data_length = txMsg.dataLen,      // 64字节

.msg_id_type = FLEXCAN_MSG_ID_STD,

.enable_brs  = true,               // 启用波特率切换

.fd_enable   = true,               // 开启CAN FD模式

.fd_padding  = 0U

};

// 3次重试机制

uint8_t retry_count = 0;

status_t send_status = STATUS_ERROR;

while (retry_count < 3 && send_status != STATUS_SUCCESS)

{

FLEXCAN_DRV_ConfigTxMb(INST_CANCOM1, CAN1_TX_MAILBOX, &fd_dataInfo, txMsg.msgId);

send_status = FLEXCAN_DRV_Send(INST_CANCOM1, CAN1_TX_MAILBOX, &fd_dataInfo, txMsg.msgId, txMsg.data);

if (send_status != STATUS_SUCCESS)

{

retry_count++;

vTaskDelay(pdMS_TO_TICKS(5));

}

}

if(send_status == STATUS_SUCCESS)

{

LOG("CAN1发送成功: ID=0x%03lX 长度=%d字节\r\n", txMsg.msgId, txMsg.dataLen);

PINS_DRV_TogglePins(LED_PORT, (1 << LED2)); // LED2闪烁指示

}

else

{

LOG("CAN1发送失败: ID=0x%03lX 重试次数=%d\r\n", txMsg.msgId, retry_count);

}

xSemaphoreGive(xCan1Mutex);

}

}

// 队列剩余空间检查

UBaseType_t queue_spaces = uxQueueSpacesAvailable(xCan1TxQueue);

if (queue_spaces < 10)

{

LOG("警告: CAN1发送队列剩余空间不足: %lu\r\n", queue_spaces);

}

Watchdog_Pet(WD_CAN1_TX);

vTaskDelay(pdMS_TO_TICKS(2));

}

}

/******************************************************************************

* UART任务（含看门狗喂狗，保留原有逻辑）

******************************************************************************/

void UART_Task(void *pvParameters)

{

flexcan_msgbuff_t uartMsg;

EventBits_t uxBits;

(void)pvParameters;

UART_ReceiveData(&uart_pal1_instance, uartRxBuffer, 1U);

LOG("UART任务启动成功 - 支持回显和CAN数据打印\r\n");

/* 任务循环：喂看门狗（WD_UART） */

for(;;)

{

const TickType_t xMaxBlockTime = pdMS_TO_TICKS(500);

uxBits = xEventGroupWaitBits(

xSystemEvents,

EVENT_CAN_DATA_RECEIVED | EVENT_UART_TX_READY,

pdTRUE,

pdFALSE,

xMaxBlockTime

);

if((uxBits & EVENT_CAN_DATA_RECEIVED) != 0)

{

if(xQueueReceive(xCanRxQueue, &uartMsg, 0) == pdTRUE)

{

if(uartMsg.dataLen > 0 && uartMsg.data[0] == 0x01)

{

PINS_DRV_TogglePins(LED_PORT, (1 << LED1));

}

}

}

if((uxBits & EVENT_UART_TX_READY) != 0 && rxComplete)

{

uartRxBuffer[bufferIdx] = '\0';

if(xSemaphoreTake(xUartMutex, pdMS_TO_TICKS(10)) == pdTRUE)

{

UART_SendDataBlocking(&uart_pal1_instance, (uint8_t*)"Echo: ", 6, TIMEOUT);

UART_SendDataBlocking(&uart_pal1_instance, uartRxBuffer, bufferIdx, TIMEOUT);

UART_SendDataBlocking(&uart_pal1_instance, (uint8_t*)"\r\n", 2, TIMEOUT);

xSemaphoreGive(xUartMutex);

}

bufferIdx = 0;

rxComplete = false;

UART_ReceiveData(&uart_pal1_instance, uartRxBuffer, 1U);

}

Watchdog_Pet(WD_UART);

vTaskDelay(pdMS_TO_TICKS(1));

}

}

/******************************************************************************

* LED任务（含看门狗喂狗，保留原有逻辑）

******************************************************************************/

void LED_Task(void *pvParameters)

{

EventBits_t uxBits;

(void)pvParameters;

for(;;)

{

// LED2作为心跳灯，每500ms翻转一次

PINS_DRV_TogglePins(LED_PORT, (1 << LED2));

/* 喂看门狗 */

Watchdog_Pet(WD_LED);

vTaskDelay(pdMS_TO_TICKS(500));

// 检测按键事件（EVENT_BUTTON1_PRESSED），触发LED1翻转

uxBits = xEventGroupGetBits(xSystemEvents);

if((uxBits & EVENT_BUTTON1_PRESSED) != 0)

{

xEventGroupClearBits(xSystemEvents, EVENT_BUTTON1_PRESSED);

PINS_DRV_TogglePins(LED_PORT, (1 << LED1));

}

}

}

/******************************************************************************

* 看门狗核心函数（修改：新增UDP任务监控）

******************************************************************************/

void Watchdog_Pet(WatchdogId_t id)

{

if (id >= WD_COUNT) return;

taskENTER_CRITICAL();

g_watchdog_last_tick[id] = xTaskGetTickCount();

taskEXIT_CRITICAL();

}

static void Watchdog_Task(void *pvParameters)

{

(void)pvParameters;

const TickType_t check_delay = pdMS_TO_TICKS(WATCHDOG_CHECK_MS);

const TickType_t timeout_ticks = pdMS_TO_TICKS(WATCHDOG_TIMEOUT_MS);

LOG("看门狗任务启动 - 监控所有%d个任务\r\n", WD_COUNT);

/* 初始化所有任务时间戳 */

static BaseType_t watchdog_initialized = pdFALSE;

if (watchdog_initialized == pdFALSE)

{

taskENTER_CRITICAL();

for (int k = 0; k < WD_COUNT; k++) g_watchdog_last_tick[k] = xTaskGetTickCount();

taskEXIT_CRITICAL();

watchdog_initialized = pdTRUE;

}

/* 看门狗循环检查（新增UDP任务名称） */

const char *task_names[WD_COUNT] = {

"CAN_Init", "SPI_Init", "UART", "LED",

"CAN2_Rx", "SPI_Tx", "SPI_Rx", "CAN1_Tx",

"UDP_Send", "ETH_Init"

};

for (;;)

{

vTaskDelay(check_delay);

TickType_t now = xTaskGetTickCount();

// 遍历监控所有任务

for (int i = 0; i < WD_COUNT; i++)

{

TickType_t last;

taskENTER_CRITICAL();

last = g_watchdog_last_tick[i];

taskEXIT_CRITICAL();

// 计算已耗时（处理Tick溢出）

TickType_t elapsed;

if (now >= last)

elapsed = now - last;

else

elapsed = (0xFFFFFFFF - last) + now;

// 超时处理

if(elapsed > timeout_ticks)

{

LOG("看门狗超时: 任务 [%s] | 耗时: %lu 嘀嗒\r\n", task_names[i], elapsed);

// 重置该任务计时器

taskENTER_CRITICAL();

g_watchdog_last_tick[i] = now;

taskEXIT_CRITICAL();

LOG("任务 [%s] 计时器已重置\r\n", task_names[i]);

}

}

}

}

/******************************************************************************

* 辅助函数（保留原有逻辑）

******************************************************************************/

void UART_PrintCANMsg(uint32_t msgId, uint8_t *data, uint8_t len)

{

char buf[64];

int pos = 0;

pos += sprintf(buf + pos, "CAN接收: ID=0x%03lX | 数据:", msgId);

for (uint8_t i = 0; i < len && pos < (int)sizeof(buf) - 4; i++)

{

pos += sprintf(buf + pos, "%02X ", data[i]);

}

pos += sprintf(buf + pos, "\r\n");

if (xUartMutex != NULL && xSemaphoreTake(xUartMutex, pdMS_TO_TICKS(10)) == pdTRUE)

{

UART_SendDataBlocking(&uart_pal1_instance, (uint8_t*)buf, (uint32_t)pos, TIMEOUT);

xSemaphoreGive(xUartMutex);

}

else

{

UART_SendDataBlocking(&uart_pal1_instance, (uint8_t*)buf, (uint32_t)pos, TIMEOUT);

}

}

void LED_ControlByCAN(uint32_t msgId, uint8_t *data, uint8_t dataLen)

{

switch (msgId) {

case CAN_LED_CTRL_ID_LED0_ON:

PINS_DRV_SetPins(LED_PORT, 1UL << LED0);

return;

case CAN_LED_CTRL_ID_LED1_ON:

PINS_DRV_SetPins(LED_PORT, 1UL << LED1);

return;

default: break;

}

if (dataLen > 0) {

switch (data[0]) {

case CAN_LED_CTRL_DATA_TOGGLE:

PINS_DRV_TogglePins(LED_PORT, (1UL << LED0) | (1UL << LED1));

break;

case CAN_LED_CTRL_DATA_OFF:

PINS_DRV_ClearPins(LED_PORT, (1UL << LED0) | (1UL << LED1));

break;

case CAN_LED_CTRL_DATA_ON:

PINS_DRV_SetPins(LED_PORT, (1UL << LED0) | (1UL << LED1));

break;

default:

PINS_DRV_TogglePins(LED_PORT, 1UL << LED0);

break;

}

}

}

void SPI_SendCANMessage(uint32_t msgId, uint8_t *data, uint8_t dataLen)

{

flexcan_msgbuff_t spiMsg;

spiMsg.msgId = msgId;

spiMsg.dataLen = dataLen;

memcpy(spiMsg.data, data, dataLen);

if (xSpiTxQueue != NULL)

{

if (xQueueSend(xSpiTxQueue, &spiMsg, 0) != pdPASS)

{

LOG("SPI队列满: 丢弃CAN消息 0x%03lX\r\n", msgId);

}

else

{

LOG("CAN→SPI: 消息 0x%03lX 已加入SPI发送队列\r\n", msgId);

}

}

}

void uart_log(const char *fmt, ...)

{

char buf[256];

va_list args;

va_start(args, fmt);

int len = vsnprintf(buf, sizeof(buf), fmt, args);

va_end(args);

if (len <= 0) return;

if (len > (int)sizeof(buf)) len = (int)sizeof(buf);

if (xTaskGetSchedulerState() == taskSCHEDULER_NOT_STARTED)

{

printf("%s", buf);

return;

}

if (xUartMutex != NULL && xSemaphoreTake(xUartMutex, pdMS_TO_TICKS(20)) == pdTRUE)

{

UART_SendDataBlocking(&uart_pal1_instance, (uint8_t*)buf, (uint32_t)len, TIMEOUT);

xSemaphoreGive(xUartMutex);

return;

}

UART_SendDataBlocking(&uart_pal1_instance, (uint8_t*)buf, (uint32_t)len, TIMEOUT);

}

void CANCOM1_Init(void)

{

LOG("CANCOM1 初始化（CAN FD模式）...\r\n");

status_t status = FLEXCAN_DRV_Init(INST_CANCOM1, &canCom1_State, &canCom1_InitConfig0);

if (status == STATUS_SUCCESS) {

LOG("CANCOM1 初始化成功（CAN FD模式）\r\n");

LOG("  支持CAN FD，最大64字节数据长度\r\n");

} else {

LOG("CANCOM1 初始化失败（状态: %d）\r\n", status);

}

}

void CANCOM2_Init(void)

{

LOG("CANCOM2 初始化（标准CAN模式）...\r\n");

status_t status = FLEXCAN_DRV_Init(INST_CANCOM2, &canCom2_State, &canCom2_InitConfig0);

if (status == STATUS_SUCCESS) {

LOG("CANCOM2 初始化成功（标准CAN模式）\r\n");

LOG("  支持标准CAN，最大8字节数据长度\r\n");

} else {

LOG("CANCOM2 初始化失败（状态: %d）\r\n", status);

}

}

void DSPI_Init(void)

{

LOG("DSPI 初始化（主机模式）...\r\n");

dspi_master_config_t spi_config;

status_t status = DSPI_GetDefaultMasterCfg(&spi_config);

if (status != STATUS_SUCCESS)

{

LOG("DSPI 获取默认配置失败! 状态: %d\r\n", status);

return;

}

spi_config.bitsPerSec = 1000000U;

spi_config.pcsPolarity = DSPI_ACTIVE_LOW;

spi_config.clkPhase = DSPI_CLOCK_PHASE_1ST_EDGE;

spi_config.clkPolarity = DSPI_ACTIVE_HIGH;

status = DSPI_MasterInit(INST_DSPI1, &dspi1State, &spi_config);

if (status != STATUS_SUCCESS)

{

LOG("DSPI 主机初始化失败! 状态: %d\r\n", status);

}

else

{

LOG("DSPI 初始化成功\r\n");

LOG("  SPI配置: 1MHz, Mode0(CPOL=0, CPHA=0), 主机模式\r\n");

}

}

void Board_Init(void)

{

CLOCK_SYS_Init(g_clockManConfigsArr, CLOCK_MANAGER_CONFIG_CNT,

g_clockManCallbacksArr, CLOCK_MANAGER_CALLBACK_CNT);

CLOCK_SYS_UpdateConfiguration(0U, CLOCK_MANAGER_POLICY_FORCIBLE);

PINS_DRV_Init(NUM_OF_CONFIGURED_PINS, g_pin_mux_InitConfigArr);

}

void GPIO_Init(void)

{

PINS_DRV_ClearPins(LED_PORT, (1 << LED0) | (1 << LED1));

PINS_DRV_SetPins(LED_PORT, (1 << LED2));

}

/******************************************************************************

* 新增：以太网相关函数实现

******************************************************************************/

static void enetif_init(void)

{

#if LWIP_IPV4

ip4_addr_t ipaddr, netmask, gw;

#endif

#if LWIP_DHCP || LWIP_AUTOIP

err_t err;

#endif

#if LWIP_IPV4

// 静态IP配置（如需DHCP，注释以下3行并启用DHCP相关代码）

IP4_ADDR(&ipaddr, 192, 168, 5, 100);  // 本地IP（可根据实际修改）

IP4_ADDR(&netmask, 255, 255, 255, 0); // 子网掩码

IP4_ADDR(&gw, 192, 168, 5, 1);        // 网关

#if LWIP_DHCP

// DHCP初始化（如需动态IP启用）

dhcp_set_struct(&netif, &netif_dhcp);

err = dhcp_start(&netif);

if (err != ERR_OK) {

LOG("DHCP启动失败: %d\r\n", err);

}

#elif LWIP_AUTOIP

// AUTOIP初始化（如需自动IP启用）

autoip_set_struct(&netif, &netif_autoip);

err = autoip_start(&netif);

if (err != ERR_OK) {

LOG("AUTOIP启动失败: %d\r\n", err);

}

#endif

#endif

// 添加网络接口并设置为默认

#if NO_SYS

netif_set_default(netif_add(&netif, &ipaddr, &netmask, &gw, NULL, enet_ethernetif_init, netif_input));

#else

netif_set_default(netif_add(&netif, &ipaddr, &netmask, &gw, NULL, enet_ethernetif_init, tcpip_input));

#endif

#if LWIP_IPV6

netif_create_ip6_linklocal_address(&netif, 1);

#endif

netif_set_up(&netif); // 启用网络接口

LOG("以太网接口初始化完成\r\n");

LOG("本地IP: %s\r\n", ip4addr_ntoa(&ipaddr));

LOG("子网掩码: %s\r\n", ip4addr_ntoa(&netmask));

LOG("网关: %s\r\n", ip4addr_ntoa(&gw));

}

static void eth_test_init(void* arg)

{

#if !NO_SYS

sys_sem_t* init_sem = (sys_sem_t*)arg;

LWIP_ASSERT("init_sem != NULL", init_sem != NULL);

#endif

enetif_init();

Watchdog_Pet(WD_ETH_INIT);

#if !NO_SYS

sys_sem_signal(init_sem);

#endif

}

/******************************************************************************

* 新增：以太网初始化任务

******************************************************************************/

void Ethernet_Init_Task(void *pvParameters)

{

(void)pvParameters;

LOG("开始以太网初始化...\r\n");

#if !NO_SYS

err_t err;

sys_sem_t init_sem;

err = sys_sem_new(&init_sem, 0);

if (err != ERR_OK) {

LOG("创建初始化信号量失败\r\n");

vTaskDelete(NULL);

return;

}

// 初始化TCP/IP协议栈

tcpip_init(eth_test_init, (void*)&init_sem);

(void)sys_sem_wait(&init_sem);

sys_sem_free(&init_sem);

#if (LWIP_SOCKET || LWIP_NETCONN) && LWIP_NETCONN_SEM_PER_THREAD

netconn_thread_init();

#endif

#else

sys_init();

lwip_init();

eth_test_init(NULL);

#endif

LOG("以太网初始化完成！\r\n");

xEventGroupSetBits(xSystemEvents, EVENT_ETH_INIT_COMPLETE);

// 任务循环：喂看门狗

for(;;) {

Watchdog_Pet(WD_ETH_INIT);

vTaskDelay(pdMS_TO_TICKS(1000));

}

}

/******************************************************************************

* 新增：UDP发送任务（核心：SPI数据UDP透传）

******************************************************************************/

static void udp_send_task(void *pvParameters)

{

struct netconn *conn = NULL;

struct netbuf *buf = NULL;

ip_addr_t target_addr;

err_t err;

(void)pvParameters;

LOG("UDP发送任务启动\r\n");

// 等待以太网初始化完成

xEventGroupWaitBits(xSystemEvents, EVENT_ETH_INIT_COMPLETE, pdFALSE, pdTRUE, portMAX_DELAY);

vTaskDelay(pdMS_TO_TICKS(2000)); // 等待网络稳定

// 解析目标IP地址

#if LWIP_IPV4

if (ip4addr_aton(UDP_TARGET_IP, ip_2_ip4(&target_addr)) == 0) {

LOG("无效的目标IP地址: %s\r\n", UDP_TARGET_IP);

vTaskDelete(NULL);

return;

}

LOG("目标IP: %s, 目标端口: %d\r\n", UDP_TARGET_IP, UDP_TARGET_PORT);

#endif

// 创建UDP连接

conn = netconn_new(NETCONN_UDP);

if (conn == NULL) {

LOG("创建UDP连接失败\r\n");

vTaskDelete(NULL);

return;

}

// 绑定本地端口

err = netconn_bind(conn, IP_ADDR_ANY, UDP_LOCAL_PORT);

if (err != ERR_OK) {

LOG("绑定本地端口 %d 失败: %d\r\n", UDP_LOCAL_PORT, err);

netconn_delete(conn);

vTaskDelete(NULL);

return;

}

// 连接到目标IP和端口

err = netconn_connect(conn, &target_addr, UDP_TARGET_PORT);

if (err != ERR_OK) {

LOG("连接到 %s:%d 失败: %d\r\n", UDP_TARGET_IP, UDP_TARGET_PORT, err);

netconn_delete(conn);

vTaskDelete(NULL);

return;

}

LOG("UDP连接建立成功，开始转发SPI数据...\r\n");

// 循环发送数据

while (1) {

SpiRawData_UDP spiUdpData;

// 从UDP队列读取SPI数据

if (xQueueReceive(xUdpSendQueue, &spiUdpData, pdMS_TO_TICKS(50)) == pdTRUE)

{

LOG("UDP发送: SPI ID=0x%03lX, 数据长度=64字节, 往返时间=%lu ms\r\n",

spiUdpData.spi_raw_id, spiUdpData.roundTripTime);

// 创建网络缓冲区

buf = netbuf_new();

if (buf == NULL) {

LOG("UDP: 创建缓冲区失败\r\n");

continue;

}

// 绑定数据到缓冲区

err = netbuf_ref(buf, &spiUdpData, sizeof(SpiRawData_UDP));

if (err != ERR_OK) {

LOG("UDP: 绑定数据失败, 错误码=%d\r\n", err);

netbuf_delete(buf);

buf = NULL;

continue;

}

// 发送UDP数据

err = netconn_send(conn, buf);

if (err != ERR_OK) {

LOG("UDP: 发送失败, 错误码=%d\r\n", err);

} else {

LOG("UDP: 发送成功, 数据大小=%d字节\r\n", sizeof(SpiRawData_UDP));

PINS_DRV_TogglePins(LED_PORT, (1 << LED0)); // LED0闪烁指示UDP发送成功

}

// 释放缓冲区

netbuf_delete(buf);

buf = NULL;

}

// 喂看门狗

Watchdog_Pet(WD_UDP_SEND);

}

// 释放资源（正常不会执行到这里）

if (buf != NULL) netbuf_delete(buf);

if (conn != NULL) netconn_delete(conn);

vTaskDelete(NULL);

}

/******************************************************************************

* RTOS基础对象初始化（修改：新增UDP队列和以太网任务）

******************************************************************************/

void RTOS_Objects_Init_Base(void)

{

/* 创建核心RTOS对象 */

xSystemEvents = xEventGroupCreate();

xUartMutex = xSemaphoreCreateMutex();

xCanMutex = xSemaphoreCreateMutex();

xSpiMutex = xSemaphoreCreateMutex();

xCan1Mutex = xSemaphoreCreateMutex();

xUdpSendQueue = xQueueCreate(20, sizeof(SpiRawData_UDP)); // 创建UDP发送队列（容量20）

/* 断言检查 */

configASSERT(xSystemEvents != NULL);

configASSERT(xUartMutex != NULL);

configASSERT(xCanMutex != NULL);

configASSERT(xSpiMutex != NULL);

configASSERT(xCan1Mutex != NULL);

configASSERT(xUdpSendQueue != NULL); // 检查UDP队列创建

/* 创建看门狗任务（优先级最高：configMAX_PRIORITIES - 1） */

BaseType_t wd_task_status = xTaskCreate(Watchdog_Task, "Watchdog", 512, NULL, configMAX_PRIORITIES - 1, NULL);

configASSERT(wd_task_status == pdPASS);

/* 创建以太网初始化任务 */

BaseType_t eth_task_status = xTaskCreate(Ethernet_Init_Task, "EthInit", 768, NULL, 3, &xEthInitTaskHandle);

configASSERT(eth_task_status == pdPASS);

LOG("RTOS基础对象初始化完成 - 含看门狗+CAN1互斥锁+UDP队列\r\n");

}

/******************************************************************************

* RTOS功能对象初始化（保留原有逻辑）

******************************************************************************/

void RTOS_Objects_Init_Func(void)

{

LOG("开始创建所有功能对象 - 队列 + CAN/SPI传输任务\r\n");

/* 创建队列 */

xCanRxQueue = xQueueCreate(5, sizeof(flexcan_msgbuff_t));

xSpiTxQueue = xQueueCreate(10, sizeof(flexcan_msgbuff_t));

xSpiRxQueue = xQueueCreate(5, sizeof(SpiRxData));

xCan1TxQueue = xQueueCreate(50, sizeof(flexcan_msgbuff_t));

/* 断言检查 */

configASSERT(xCanRxQueue != NULL);

configASSERT(xSpiTxQueue != NULL);

configASSERT(xSpiRxQueue != NULL);

configASSERT(xCan1TxQueue != NULL);

/* 创建任务（CAN1_Tx栈设为512，解决内存不足问题） */

BaseType_t taskCreateStatus;

taskCreateStatus = xTaskCreate(CAN2_Rx_Task, "CAN2_Rx", 512, NULL, 3, &xCan2RxTaskHandle);

configASSERT(taskCreateStatus == pdPASS);

taskCreateStatus = xTaskCreate(SPI_Tx_Task, "SPI_Tx", 512, NULL, 3, &xSpiTxTaskHandle);

configASSERT(taskCreateStatus == pdPASS);

taskCreateStatus = xTaskCreate(SPI_Rx_Task, "SPI_Rx", 512, NULL, 3, &xSpiRxTaskHandle);

configASSERT(taskCreateStatus == pdPASS);

taskCreateStatus = xTaskCreate(CAN1_Tx_Task, "CAN1_Tx", 512, NULL, 2, &xCan1TxTaskHandle);

configASSERT(taskCreateStatus == pdPASS);

xEventGroupSetBits(xSystemEvents, EVENT_FUNC_STARTED);

LOG("所有功能对象创建完成 - 含CAN1 CAN FD发送任务\r\n");

}

/******************************************************************************

* 主函数（修改：新增UDP发送任务创建）

******************************************************************************/

volatile int exit_code = 0;

int main(void)

{

#ifdef PEX_RTOS_INIT

PEX_RTOS_INIT();

#endif

srand(0x1234);

/* 硬件底层初始化 */

Board_Init();

GPIO_Init();

UART_Init(&uart_pal1_instance, &uart_pal1_Config0);

UART_ReceiveData(&uart_pal1_instance, uartRxBuffer, 1U);

/* 打印启动信息 */

LOG("\r\n");

LOG("================================================\r\n");

LOG("多外设系统启动（最终完整版：含看门狗+CAN FD+UDP转发）\r\n");

LOG("核心功能：CAN2→SPI + SPI→CAN1 64字节转发 + SPI→UDP 64字节转发 + 10任务看门狗监控\r\n");

LOG("系统编译时间: %s %s\r\n", __DATE__, __TIME__);

LOG("================================================\r\n");

/* RTOS基础对象初始化（含看门狗+UDP队列+以太网任务） */

RTOS_Objects_Init_Base();

/* 创建基础任务 */

xTaskCreate(CAN_Init_Task, "CAN Init", 512, NULL, 3, &xCanInitTaskHandle);

xTaskCreate(SPI_Init_Task, "SPI Init", 512, NULL, 3, &xSpiInitTaskHandle);

xTaskCreate(UART_Task, "UART", 512, NULL, 2, &xUartTaskHandle);

xTaskCreate(LED_Task, "LED", 128, NULL, 1, &xLedTaskHandle);

/* 新增：创建UDP发送任务 */

BaseType_t udp_task_status = xTaskCreate(udp_send_task, "UDP_Send", 1536, NULL, UDP_SEND_TASK_PRIORITY, &xUdpSendTaskHandle);

configASSERT(udp_task_status == pdPASS);

LOG("UDP发送任务创建成功\r\n");

/* 启动FreeRTOS调度器 */

LOG("启动FreeRTOS调度器...\r\n");

vTaskStartScheduler();

/* 调度器启动失败：LED全闪报警 */

LOG("调度器启动失败!\r\n");

for(;;)

{

PINS_DRV_TogglePins(LED_PORT, (1 << LED0) | (1 << LED1) | (1 << LED2));

volatile uint32_t i;

for(i = 0; i < 500000; i++);

}

return exit_code;

}