# -*- coding: UTF-8 -*-

import socket

import struct

from datetime import datetime

# ==================== 配置项（和MCU代码完全匹配） ====================

UDP_LISTEN_PORT = 8089  # 和MCU的UDP_TARGET_PORT一致

UDP_BUFFER_SIZE = 1024  # 缓冲区足够大即可

# 结构体数据长度定义 (和MCU的CanRawData_UDP对应)

CAN_ID_BYTE_LEN = 4  # can_raw_id  占4字节

CAN_DATA_BYTE_LEN = 64  # can_data[]  占64字节 【核心64字节】

J1939_FLAG_LEN = 1  # is_j1939    占1字节

FILL_BYTE_LEN = 3  # 内存对齐填充字节 占3字节

TOTAL_DATA_LEN = 72  # 4+64+1+3 = 72 字节 (固定值)

# ==================== 车速/转速解析全局变量（保存上次有效值） ====================

last_valid_rpm = None

last_valid_speed = None

# ==================== AES+HMAC核心常量（与文档/MCU完全一致） ====================

AES128_KEY = 0x123456789ABCDEF0123456789ABCDEF0

HMAC_KEY = 0x00112233445566778899aabbccddeeff00112233445566778899aabbccddeeff

# ==================== AES核心定义（文档原版） ====================

SBOX = [

0x63, 0x7c, 0x77, 0x7b, 0xf2, 0x6b, 0x6f, 0xc5, 0x30, 0x01, 0x67, 0x2b, 0xfe, 0xd7, 0xab, 0x76,

0xca, 0x82, 0xc9, 0x7d, 0xfa, 0x59, 0x47, 0xf0, 0xad, 0xd4, 0xa2, 0xaf, 0x9c, 0xa4, 0x72, 0xc0,

0xb7, 0xfd, 0x93, 0x26, 0x36, 0x3f, 0xf7, 0xcc, 0x34, 0xa5, 0xe5, 0xf1, 0x71, 0xd8, 0x31, 0x15,

0x04, 0xc7, 0x23, 0xc3, 0x18, 0x96, 0x05, 0x9a, 0x07, 0x12, 0x80, 0xe2, 0xeb, 0x27, 0xb2, 0x75,

0x09, 0x83, 0x2c, 0x1a, 0x1b, 0x6e, 0x5a, 0xa0, 0x52, 0x3b, 0xd6, 0xb3, 0x29, 0xe3, 0x2f, 0x84,

0x53, 0xd1, 0x00, 0xed, 0x20, 0xfc, 0xb1, 0x5b, 0x6a, 0xcb, 0xbe, 0x39, 0x4a, 0x4c, 0x58, 0xcf,

0xd0, 0xef, 0xaa, 0xfb, 0x43, 0x4d, 0x33, 0x85, 0x45, 0xf9, 0x02, 0x7f, 0x50, 0x3c, 0x9f, 0xa8,

0x51, 0xa3, 0x40, 0x8f, 0x92, 0x9d, 0x38, 0xf5, 0xbc, 0xb6, 0xda, 0x21, 0x10, 0xff, 0xf3, 0xd2,

0xcd, 0x0c, 0x13, 0xec, 0x5f, 0x97, 0x44, 0x17, 0xc4, 0xa7, 0x7e, 0x3d, 0x64, 0x5d, 0x19, 0x73,

0x60, 0x81, 0x4f, 0xdc, 0x22, 0x2a, 0x90, 0x88, 0x46, 0xee, 0xb8, 0x14, 0xde, 0x5e, 0x0b, 0xdb,

0xe0, 0x32, 0x3a, 0x0a, 0x49, 0x06, 0x24, 0x5c, 0xc2, 0xd3, 0xac, 0x62, 0x91, 0x95, 0xe4, 0x79,

0xe7, 0xc8, 0x37, 0x6d, 0x8d, 0xd5, 0x4e, 0xa9, 0x6c, 0x56, 0xf4, 0xea, 0x65, 0x7a, 0xae, 0x08,

0xba, 0x78, 0x25, 0x2e, 0x1c, 0xa6, 0xb4, 0xc6, 0xe8, 0xdd, 0x74, 0x1f, 0x4b, 0xbd, 0x8b, 0x8a,

0x70, 0x3e, 0xb5, 0x66, 0x48, 0x03, 0xf6, 0x0e, 0x61, 0x35, 0x57, 0xb9, 0x86, 0xc1, 0x1d, 0x9e,

0xe1, 0xf8, 0x98, 0x11, 0x69, 0xd9, 0x8e, 0x94, 0x9b, 0x1e, 0x87, 0xe9, 0xce, 0x55, 0x28, 0xdf,

0x8c, 0xa1, 0x89, 0x0d, 0xbf, 0xe6, 0x42, 0x68, 0x41, 0x99, 0x2d, 0x0f, 0xb0, 0x54, 0xbb, 0x16

]

# 逆S盒计算（文档原版）

RSBOX = [0] * 256

for i in range(256):

RSBOX[SBOX[i]] = i

RCON = [0x01000000, 0x02000000, 0x04000000, 0x08000000, 0x10000000,

0x20000000, 0x40000000, 0x80000000, 0x1b000000, 0x36000000]

# ==================== 基础转换工具（文档原版） ====================

def bytes_to_list(data, length):

return [(data >> (i * 8)) & 0xFF for i in range(length - 1, -1, -1)]

def list_to_bytes(data_list):

res = 0

for b in data_list:

res = (res << 8) | b

return res

def format_hex_data(data):

"""格式化64字节数据为每行16字节"""

lines = []

for i in range(0, 64, 16):

lines.append(" ".join([f"{byte:02X}" for byte in data[i:i + 16]]))

return lines

# ==================== AES解密核心运算（文档原版） ====================

def inv_sub_bytes(state):

sb = bytes_to_list(state, 16)

return list_to_bytes([RSBOX[b] for b in sb])

def inv_shift_rows(state):

s = bytes_to_list(state, 16)

# 逆行移位：右移

s[1], s[5], s[9], s[13] = s[13], s[1], s[5], s[9]

s[2], s[6], s[10], s[14] = s[10], s[14], s[2], s[6]

s[3], s[7], s[11], s[15] = s[7], s[11], s[15], s[3]

return list_to_bytes(s)

def mul_gf(a, b):

p = 0

for _ in range(8):

if b & 1:

p ^= a

hi = a & 0x80

a = (a << 1) & 0xFF

if hi:

a ^= 0x1B

b >>= 1

return p

def inv_mix_columns(state):

s = bytes_to_list(state, 16)

ns = [0] * 16

for c in range(4):

c0, c1, c2, c3 = s[c * 4], s[c * 4 + 1], s[c * 4 + 2], s[c * 4 + 3]

ns[c * 4] = mul_gf(c0, 0x0e) ^ mul_gf(c1, 0x0b) ^ mul_gf(c2, 0x0d) ^ mul_gf(c3, 0x09)

ns[c * 4 + 1] = mul_gf(c0, 0x09) ^ mul_gf(c1, 0x0e) ^ mul_gf(c2, 0x0b) ^ mul_gf(c3, 0x0d)

ns[c * 4 + 2] = mul_gf(c0, 0x0d) ^ mul_gf(c1, 0x09) ^ mul_gf(c2, 0x0e) ^ mul_gf(c3, 0x0b)

ns[c * 4 + 3] = mul_gf(c0, 0x0b) ^ mul_gf(c1, 0x0d) ^ mul_gf(c2, 0x09) ^ mul_gf(c3, 0x0e)

return list_to_bytes(ns)

# ==================== 密钥扩展与解密主函数（文档原版） ====================

def rot_word(word):

return ((word << 8) | (word >> 24)) & 0xFFFFFFFF

def sub_word(word):

res = 0

for i in range(4):

byte = (word >> (24 - i * 8)) & 0xFF

res = (res << 8) | SBOX[byte]

return res

def key_expansion(key):

rk = [0] * 11

rk[0] = key

for i in range(1, 11):

temp = sub_word(rot_word(rk[i - 1] & 0xFFFFFFFF))

w0 = ((rk[i - 1] >> 96) & 0xFFFFFFFF) ^ temp ^ RCON[i - 1]

w1 = w0 ^ ((rk[i - 1] >> 64) & 0xFFFFFFFF)

w2 = w1 ^ ((rk[i - 1] >> 32) & 0xFFFFFFFF)

w3 = w2 ^ (rk[i - 1] & 0xFFFFFFFF)

rk[i] = (w0 << 96) | (w1 << 64) | (w2 << 32) | w3

return rk

def aes_decrypt(ciphertext, key):

rk = key_expansion(key)

state = ciphertext ^ rk[10]

for r in range(9, -1, -1):

state = inv_shift_rows(state)

state = inv_sub_bytes(state)

state ^= rk[r]

if r > 0:

state = inv_mix_columns(state)

return state

# ==================== SHA256 & HMAC（文档原版） ====================

SHA256_K = [

0x428a2f98, 0x71374491, 0xb5c0fbcf, 0xe9b5dba5, 0x3956c25b, 0x59f111f1, 0x923f82a4, 0xab1c5ed5,

0xd807aa98, 0x12835b01, 0x243185be, 0x550c7dc3, 0x72be5d74, 0x80deb1fe, 0x9bdc06a7, 0xc19bf174,

0xe49b69c1, 0xefbe4786, 0x0fc19dc6, 0x240ca1cc, 0x2de92c6f, 0x4a7484aa, 0x5cb0a9dc, 0x76f988da,

0x983e5152, 0xa831c66d, 0xb00327c8, 0xbf597fc7, 0xc6e00bf3, 0xd5a79147, 0x06ca6351, 0x14292967,

0x27b70a85, 0x2e1b2138, 0x4d2c6dfc, 0x53380d13, 0x650a7354, 0x766a0abb, 0x81c2c92e, 0x92722c85,

0xa2bfe8a1, 0xa81a664b, 0xc24b8b70, 0xc76c51a3, 0xd192e819, 0xd6990624, 0xf40e3585, 0x106aa070,

0x19a4c116, 0x1e376c08, 0x2748774c, 0x34b0bcb5, 0x391c0cb3, 0x4ed8aa4a, 0x5b9cca4f, 0x682e6ff3,

0x748f82ee, 0x78a5636f, 0x84c87814, 0x8cc70208, 0x90befffa, 0xa4506ceb, 0xbef9a3f7, 0xc67178f2

]

SHA256_INIT = [0x6a09e667, 0xbb67ae85, 0x3c6ef372, 0xa54ff53a, 0x510e527f, 0x9b05688c, 0x1f83d9ab, 0x5be0cd19]

def sigma0(x):

return ((x >> 7) | (x << 25)) & 0xFFFFFFFF ^ ((x >> 18) | (x << 14)) & 0xFFFFFFFF ^ (x >> 3)

def sigma1(x):

return ((x >> 17) | (x << 15)) & 0xFFFFFFFF ^ ((x >> 19) | (x << 13)) & 0xFFFFFFFF ^ (x >> 10)

def Sigma0(x):

return ((x >> 2) | (x << 30)) & 0xFFFFFFFF ^ ((x >> 13) | (x << 19)) & 0xFFFFFFFF ^ (

(x >> 22) | (x << 10)) & 0xFFFFFFFF

def Sigma1(x):

return ((x >> 6) | (x << 26)) & 0xFFFFFFFF ^ ((x >> 11) | (x << 21)) & 0xFFFFFFFF ^ (

(x >> 25) | (x << 7)) & 0xFFFFFFFF

def Ch(x, y, z):

return (x & y) ^ (~x & z)

def Maj(x, y, z):

return (x & y) ^ (x & z) ^ (y & z)

def sha256_core(msg_block, init_hash=None):

h = SHA256_INIT[:] if init_hash is None else [init_hash[k] for k in 'abcdefgh']

a, b, c, d, e, f, g, h_reg = h

w = [0] * 64

for i in range(16):

w[i] = (msg_block >> (i * 32)) & 0xFFFFFFFF

for t in range(1, 65):

if t >= 17:

w[t - 1] = (sigma1(w[t - 3]) + sigma0(w[t - 16]) + w[t - 8] + w[t - 17]) & 0xFFFFFFFF

T1 = (h_reg + Sigma1(e) + Ch(e, f, g) + SHA256_K[t - 1] + w[t - 1]) & 0xFFFFFFFF

T2 = (Sigma0(a) + Maj(a, b, c)) & 0xFFFFFFFF

a, b, c, d, e, f, g, h_reg = (T1 + T2) & 0xFFFFFFFF, a, b, c, (d + T1) & 0xFFFFFFFF, e, f, g

acc = [(x + y) & 0xFFFFFFFF for x, y in zip([a, b, c, d, e, f, g, h_reg], h)]

final_hash = 0

for word in acc:

final_hash = (final_hash << 32) | word

return {'acc_regs': dict(zip('abcdefgh', acc)), 'final_hash': final_hash}

def hmac_sha256(msg, key):

k_pad = (key << 256) & ((1 << 512) - 1)

k_ipad = k_pad ^ int.from_bytes(b'\x36' * 64, 'big')

k_opad = k_pad ^ int.from_bytes(b'\x5c' * 64, 'big')

inner1 = sha256_core(k_ipad)

msg_pad = (msg << 384) | (1 << 383) | 128

inner2 = sha256_core(msg_pad & ((1 << 512) - 1), init_hash=inner1['acc_regs'])

outer1 = sha256_core(k_opad)

outer2 = sha256_core(((inner2['final_hash'] << 256) | (1 << 255) | 256) & ((1 << 512) - 1),

init_hash=outer1['acc_regs'])

return outer2['final_hash']

# ==================== 车速/转速解析核心函数 ====================

def parse_engine_rpm(can_data_list):

"""解析发动机转速"""

if len(can_data_list) < 8:

return None

if can_data_list[0] == 0x0C and can_data_list[1] == 0x7D and can_data_list[6] == 0xF3 and can_data_list[7] == 0x7D:

rpm_raw = (can_data_list[4] << 8) | can_data_list[3]

real_rpm = rpm_raw / 8.0

return round(real_rpm, 1)

return None

def parse_vehicle_speed(can_data_list):

"""解析车辆车速"""

if len(can_data_list) < 3:

return None

speed_raw = (can_data_list[2] << 8) | can_data_list[1]

real_speed = speed_raw * 0.00390625

return round(real_speed, 4)

# ==================== UDP接收+解密+解析主函数 ====================

def udp_receive_decrypt_parse():

global last_valid_rpm, last_valid_speed

udp_socket = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

try:

udp_socket.bind(("", UDP_LISTEN_PORT))

except OSError as e:

if e.errno == 10048:

print(f"\n❌ 端口 {UDP_LISTEN_PORT} 被占用！请关闭网络调试助手/其他程序后重试\n")

return

else:

print(f"\n❌ UDP绑定失败：{e}\n")

return

# 打印启动提示

print(f"✅ CAN原始数据监听已启动 | UDP端口: {UDP_LISTEN_PORT}")

print(f"✅ 数据格式：4字节CAN_ID + 64字节核心数据 + 1字节标记 + 3字节填充 = 72字节")

print(f"✅ 验证逻辑：HMAC-SHA256完整性校验 + AES-128解密")

print("=" * 100)

try:

while True:

# 接收二进制数据

recv_data, sender_addr = udp_socket.recvfrom(UDP_BUFFER_SIZE)

# 只处理标准72字节数据

if len(recv_data) == TOTAL_DATA_LEN:

# 1. 提取核心64字节数据

can_64byte_data = recv_data[4:4 + CAN_DATA_BYTE_LEN]

# 2. 按要求打印接收到的64字节数据

print("\n【接收到64字节数据】:")

for line in format_hex_data(can_64byte_data):

print(line)

# 3. 解析64字节数据（文档原版逻辑）

header = can_64byte_data[0:5]

aes_cipher_bytes = can_64byte_data[5:21]

aes_cipher = list_to_bytes(aes_cipher_bytes)

received_hmac = list_to_bytes(can_64byte_data[21:53])

# 4. HMAC验证

print("\n【HMAC验证】")

calc_hmac = hmac_sha256(aes_cipher, HMAC_KEY)

if calc_hmac != received_hmac:

print("验证失败！数据完整性受损，跳过解密")

print("=" * 100)

continue

print("验证成功！开始解密...")

# 5. AES解密（文档原版逻辑）

plain_int = aes_decrypt(aes_cipher, AES128_KEY)

plain_bytes = bytes_to_list(plain_int, 16)

# 提取有效8字节数据

valid_plain = plain_bytes[0:8]

# 构造13字节解密输出

decrypted_output = list(header) + valid_plain

# 6. 打印解密输出数据

print("\n【解密输出数据】")

print(' '.join([f"{b:02X}" for b in decrypted_output]))

print("=" * 50)

# 7. 车速/转速解析

current_rpm = parse_engine_rpm(valid_plain)

current_speed = parse_vehicle_speed(valid_plain)

# 更新有效数据

if current_rpm is not None:

last_valid_rpm = current_rpm

if current_speed is not None:

last_valid_speed = current_speed

# 展示解析结果

show_rpm = last_valid_rpm if last_valid_rpm is not None else "暂无数据"

show_speed = last_valid_speed if last_valid_speed is not None else "暂无数据"

print(f"【解析结果】转速 = {show_rpm} RPM | 车速 = {show_speed} km/h")

print("=" * 50)

print("=" * 100)

except KeyboardInterrupt:

print("\n\n🛑 用户手动终止程序，停止监听CAN数据")

finally:

udp_socket.close()

print("✅ UDP套接字已正常关闭")

if __name__ == "__main__":

udp_receive_decrypt_parse()