`timescale 1ns/1ns

// #########################################################################

// 顶层模块：jiami（匹配原有接口，负责调度标准AES和HMAC）

// #########################################################################

module jiami(

input           sys_clk,        // 系统时钟 50/100MHz均可

input           sys_rst_n,      // 系统复位，低有效

input  [511:0]  din,            // 64字节输入 511:0

input           frame_done,     // 帧完成标志，拉高触发加密

output [511:0]  dout            // 64字节输出 固定长度，空位补0xFF

);

//=========================================================================

// 【用户可自定义 - 核心密钥区】

//=========================================================================

parameter  AES128_KEY  = 128'h123456789ABCDEF0123456789ABCDEF0;

parameter  HMAC256_KEY = 256'h00112233445566778899AABBCCDDEEFF00112233445566778899AABBCCDDEEFF;

//=========================================================================

// 内部寄存器与线网定义

//=========================================================================

reg [511:0] dout_reg;

reg [511:0] dout_reg_d1;  // 输出打拍寄存器，提升时序裕量

wire [127:0] aes_plain_data;

wire [127:0] aes_cipher;

wire         aes_done;

wire [255:0] hmac_sign;

wire         hmac_done;

integer i;

//=========================================================================

// 1. 提取8字节CAN数据 + 补8个0xFF → 凑齐16字节AES128标准明文输入

//=========================================================================

assign aes_plain_data = {din[5*8+:8], din[6*8+:8], din[7*8+:8], din[8*8+:8],

din[9*8+:8], din[10*8+:8], din[11*8+:8], din[12*8+:8],

64'hFFFFFFFFFFFFFFFF};

//=========================================================================

// 2. 例化标准AES-128模块

//=========================================================================

aes_128_standard u_aes(

.sys_clk(sys_clk),

.sys_rst_n(sys_rst_n),

.plaintext(aes_plain_data),

.key(AES128_KEY),

.start(frame_done),

.ciphertext(aes_cipher),

.done(aes_done)

);

//=========================================================================

// 3. 例化串行化标准HMAC-SHA256模块

//=========================================================================

hmac_sha256_standard u_hmac(

.sys_clk(sys_clk),

.sys_rst_n(sys_rst_n),

.msg(aes_cipher),       // 输入为AES密文（128位）

.key(HMAC256_KEY),

.start(aes_done),       // AES完成即触发HMAC

.hmac(hmac_sign),

.done(hmac_done)

);

//=========================================================================

// 4. 数据映射与输出填充 (保留原有格式，增加打拍)

//=========================================================================

assign dout = dout_reg_d1;

always @(posedge sys_clk or negedge sys_rst_n) begin

if(!sys_rst_n) begin

dout_reg    <= {512{1'b1}};

dout_reg_d1 <= {512{1'b1}};

end else if(hmac_done) begin

// 字节0~4：原样回传

dout_reg[0*8 +:8] <= din[0*8 +:8];

dout_reg[1*8 +:8] <= din[1*8 +:8];

dout_reg[2*8 +:8] <= din[2*8 +:8];

dout_reg[3*8 +:8] <= din[3*8 +:8];

dout_reg[4*8 +:8] <= din[4*8 +:8];

// 字节5~20：AES密文

dout_reg[5*8 +:8]  <= aes_cipher[127:120];

dout_reg[6*8 +:8]  <= aes_cipher[119:112];

dout_reg[7*8 +:8]  <= aes_cipher[111:104];

dout_reg[8*8 +:8]  <= aes_cipher[103:96];

dout_reg[9*8 +:8]  <= aes_cipher[95:88];

dout_reg[10*8 +:8] <= aes_cipher[87:80];

dout_reg[11*8 +:8] <= aes_cipher[79:72];

dout_reg[12*8 +:8] <= aes_cipher[71:64];

dout_reg[13*8 +:8] <= aes_cipher[63:56];

dout_reg[14*8 +:8] <= aes_cipher[55:48];

dout_reg[15*8 +:8] <= aes_cipher[47:40];

dout_reg[16*8 +:8] <= aes_cipher[39:32];

dout_reg[17*8 +:8] <= aes_cipher[31:24];

dout_reg[18*8 +:8] <= aes_cipher[23:16];

dout_reg[19*8 +:8] <= aes_cipher[15:8];

dout_reg[20*8 +:8] <= aes_cipher[7:0];

// 字节21~52：HMAC签名

dout_reg[21*8 +:8] <= hmac_sign[255:248];

dout_reg[22*8 +:8] <= hmac_sign[247:240];

dout_reg[23*8 +:8] <= hmac_sign[239:232];

dout_reg[24*8 +:8] <= hmac_sign[231:224];

dout_reg[25*8 +:8] <= hmac_sign[223:216];

dout_reg[26*8 +:8] <= hmac_sign[215:208];

dout_reg[27*8 +:8] <= hmac_sign[207:200];

dout_reg[28*8 +:8] <= hmac_sign[199:192];

dout_reg[29*8 +:8] <= hmac_sign[191:184];

dout_reg[30*8 +:8] <= hmac_sign[183:176];

dout_reg[31*8 +:8] <= hmac_sign[175:168];

dout_reg[32*8 +:8] <= hmac_sign[167:160];

dout_reg[33*8 +:8] <= hmac_sign[159:152];

dout_reg[34*8 +:8] <= hmac_sign[151:144];

dout_reg[35*8 +:8] <= hmac_sign[143:136];

dout_reg[36*8 +:8] <= hmac_sign[135:128];

dout_reg[37*8 +:8] <= hmac_sign[127:120];

dout_reg[38*8 +:8] <= hmac_sign[119:112];

dout_reg[39*8 +:8] <= hmac_sign[111:104];

dout_reg[40*8 +:8] <= hmac_sign[103:96];

dout_reg[41*8 +:8] <= hmac_sign[95:88];

dout_reg[42*8 +:8] <= hmac_sign[87:80];

dout_reg[43*8 +:8] <= hmac_sign[79:72];

dout_reg[44*8 +:8] <= hmac_sign[71:64];

dout_reg[45*8 +:8] <= hmac_sign[63:56];

dout_reg[46*8 +:8] <= hmac_sign[55:48];

dout_reg[47*8 +:8] <= hmac_sign[47:40];

dout_reg[48*8 +:8] <= hmac_sign[39:32];

dout_reg[49*8 +:8] <= hmac_sign[31:24];

dout_reg[50*8 +:8] <= hmac_sign[23:16];

dout_reg[51*8 +:8] <= hmac_sign[15:8];

dout_reg[52*8 +:8] <= hmac_sign[7:0];

// 字节53~63：补0xFF

for(i=53; i<64; i=i+1) dout_reg[i*8 +:8] <= 8'hFF;

end else begin

// 输出打拍，提升时序稳定性

dout_reg_d1 <= dout_reg;

end

end

endmodule

// #########################################################################

// 子模块：标准AES-128加密

// #########################################################################

// #########################################################################

// 子模块：面积优化版 AES-128 (折叠架构，节省 75% S-box 资源)

// #########################################################################

module aes_128_standard(

input sys_clk,

input sys_rst_n,

input [127:0] plaintext,

input [127:0] key,

input start,

output reg [127:0] ciphertext,

output reg done

);

// 标准S盒

reg [7:0] sbox[0:255];

initial begin

sbox[8'h00]=8'h63; sbox[8'h01]=8'h7c; sbox[8'h02]=8'h77; sbox[8'h03]=8'h7b;

sbox[8'h04]=8'hf2; sbox[8'h05]=8'h6b; sbox[8'h06]=8'h6f; sbox[8'h07]=8'hc5;

sbox[8'h08]=8'h30; sbox[8'h09]=8'h01; sbox[8'h0a]=8'h67; sbox[8'h0b]=8'h2b;

sbox[8'h0c]=8'hfe; sbox[8'h0d]=8'hd7; sbox[8'h0e]=8'hab; sbox[8'h0f]=8'h76;

sbox[8'h10]=8'hca; sbox[8'h11]=8'h82; sbox[8'h12]=8'hc9; sbox[8'h13]=8'h7d;

sbox[8'h14]=8'hfa; sbox[8'h15]=8'h59; sbox[8'h16]=8'h47; sbox[8'h17]=8'hf0;

sbox[8'h18]=8'had; sbox[8'h19]=8'hd4; sbox[8'h1a]=8'ha2; sbox[8'h1b]=8'haf;

sbox[8'h1c]=8'h9c; sbox[8'h1d]=8'ha4; sbox[8'h1e]=8'h72; sbox[8'h1f]=8'hc0;

sbox[8'h20]=8'hb7; sbox[8'h21]=8'hfd; sbox[8'h22]=8'h93; sbox[8'h23]=8'h26;

sbox[8'h24]=8'h36; sbox[8'h25]=8'h3f; sbox[8'h26]=8'hf7; sbox[8'h27]=8'hcc;

sbox[8'h28]=8'h34; sbox[8'h29]=8'ha5; sbox[8'h2a]=8'he5; sbox[8'h2b]=8'hf1;

sbox[8'h2c]=8'h71; sbox[8'h2d]=8'hd8; sbox[8'h2e]=8'h31; sbox[8'h2f]=8'h15;

sbox[8'h30]=8'h04; sbox[8'h31]=8'hc7; sbox[8'h32]=8'h23; sbox[8'h33]=8'hc3;

sbox[8'h34]=8'h18; sbox[8'h35]=8'h96; sbox[8'h36]=8'h05; sbox[8'h37]=8'h9a;

sbox[8'h38]=8'h07; sbox[8'h39]=8'h12; sbox[8'h3a]=8'h80; sbox[8'h3b]=8'he2;

sbox[8'h3c]=8'heb; sbox[8'h3d]=8'h27; sbox[8'h3e]=8'hb2; sbox[8'h3f]=8'h75;

sbox[8'h40]=8'h09; sbox[8'h41]=8'h83; sbox[8'h42]=8'h2c; sbox[8'h43]=8'h1a;

sbox[8'h44]=8'h1b; sbox[8'h45]=8'h6e; sbox[8'h46]=8'h5a; sbox[8'h47]=8'ha0;

sbox[8'h48]=8'h52; sbox[8'h49]=8'h3b; sbox[8'h4a]=8'hd6; sbox[8'h4b]=8'hb3;

sbox[8'h4c]=8'h29; sbox[8'h4d]=8'he3; sbox[8'h4e]=8'h2f; sbox[8'h4f]=8'h84;

sbox[8'h50]=8'h53; sbox[8'h51]=8'hd1; sbox[8'h52]=8'h00; sbox[8'h53]=8'hed;

sbox[8'h54]=8'h20; sbox[8'h55]=8'hfc; sbox[8'h56]=8'hb1; sbox[8'h57]=8'h5b;

sbox[8'h58]=8'h6a; sbox[8'h59]=8'hcb; sbox[8'h5a]=8'hbe; sbox[8'h5b]=8'h39;

sbox[8'h5c]=8'h4a; sbox[8'h5d]=8'h4c; sbox[8'h5e]=8'h58; sbox[8'h5f]=8'hcf;

sbox[8'h60]=8'hd0; sbox[8'h61]=8'hef; sbox[8'h62]=8'haa; sbox[8'h63]=8'hfb;

sbox[8'h64]=8'h43; sbox[8'h65]=8'h4d; sbox[8'h66]=8'h33; sbox[8'h67]=8'h85;

sbox[8'h68]=8'h45; sbox[8'h69]=8'hf9; sbox[8'h6a]=8'h02; sbox[8'h6b]=8'h7f;

sbox[8'h6c]=8'h50; sbox[8'h6d]=8'h3c; sbox[8'h6e]=8'h9f; sbox[8'h6f]=8'ha8;

sbox[8'h70]=8'h51; sbox[8'h71]=8'ha3; sbox[8'h72]=8'h40; sbox[8'h73]=8'h8f;

sbox[8'h74]=8'h92; sbox[8'h75]=8'h9d; sbox[8'h76]=8'h38; sbox[8'h77]=8'hf5;

sbox[8'h78]=8'hbc; sbox[8'h79]=8'hb6; sbox[8'h7a]=8'hda; sbox[8'h7b]=8'h21;

sbox[8'h7c]=8'h10; sbox[8'h7d]=8'hff; sbox[8'h7e]=8'hf3; sbox[8'h7f]=8'hd2;

sbox[8'h80]=8'hcd; sbox[8'h81]=8'h0c; sbox[8'h82]=8'h13; sbox[8'h83]=8'hec;

sbox[8'h84]=8'h5f; sbox[8'h85]=8'h97; sbox[8'h86]=8'h44; sbox[8'h87]=8'h17;

sbox[8'h88]=8'hc4; sbox[8'h89]=8'ha7; sbox[8'h8a]=8'h7e; sbox[8'h8b]=8'h3d;

sbox[8'h8c]=8'h64; sbox[8'h8d]=8'h5d; sbox[8'h8e]=8'h19; sbox[8'h8f]=8'h73;

sbox[8'h90]=8'h60; sbox[8'h91]=8'h81; sbox[8'h92]=8'h4f; sbox[8'h93]=8'hdc;

sbox[8'h94]=8'h22; sbox[8'h95]=8'h2a; sbox[8'h96]=8'h90; sbox[8'h97]=8'h88;

sbox[8'h98]=8'h46; sbox[8'h99]=8'hee; sbox[8'h9a]=8'hb8; sbox[8'h9b]=8'h14;

sbox[8'h9c]=8'hde; sbox[8'h9d]=8'h5e; sbox[8'h9e]=8'h0b; sbox[8'h9f]=8'hdb;

sbox[8'ha0]=8'he0; sbox[8'ha1]=8'h32; sbox[8'ha2]=8'h3a; sbox[8'ha3]=8'h0a;

sbox[8'ha4]=8'h49; sbox[8'ha5]=8'h06; sbox[8'ha6]=8'h24; sbox[8'ha7]=8'h5c;

sbox[8'ha8]=8'hc2; sbox[8'ha9]=8'hd3; sbox[8'haa]=8'hac; sbox[8'hab]=8'h62;

sbox[8'hac]=8'h91; sbox[8'had]=8'h95; sbox[8'hae]=8'he4; sbox[8'haf]=8'h79;

sbox[8'hb0]=8'he7; sbox[8'hb1]=8'hc8; sbox[8'hb2]=8'h37; sbox[8'hb3]=8'h6d;

sbox[8'hb4]=8'h8d; sbox[8'hb5]=8'hd5; sbox[8'hb6]=8'h4e; sbox[8'hb7]=8'ha9;

sbox[8'hb8]=8'h6c; sbox[8'hb9]=8'h56; sbox[8'hba]=8'hf4; sbox[8'hbb]=8'hea;

sbox[8'hbc]=8'h65; sbox[8'hbd]=8'h7a; sbox[8'hbe]=8'hae; sbox[8'hbf]=8'h08;

sbox[8'hc0]=8'hba; sbox[8'hc1]=8'h78; sbox[8'hc2]=8'h25; sbox[8'hc3]=8'h2e;

sbox[8'hc4]=8'h1c; sbox[8'hc5]=8'ha6; sbox[8'hc6]=8'hb4; sbox[8'hc7]=8'hc6;

sbox[8'hc8]=8'he8; sbox[8'hc9]=8'hdd; sbox[8'hca]=8'h74; sbox[8'hcb]=8'h1f;

sbox[8'hcc]=8'h4b; sbox[8'hcd]=8'hbd; sbox[8'hce]=8'h8b; sbox[8'hcf]=8'h8a;

sbox[8'hd0]=8'h70; sbox[8'hd1]=8'h3e; sbox[8'hd2]=8'hb5; sbox[8'hd3]=8'h66;

sbox[8'hd4]=8'h48; sbox[8'hd5]=8'h03; sbox[8'hd6]=8'hf6; sbox[8'hd7]=8'h0e;

sbox[8'hd8]=8'h61; sbox[8'hd9]=8'h35; sbox[8'hda]=8'h57; sbox[8'hdb]=8'hb9;

sbox[8'hdc]=8'h86; sbox[8'hdd]=8'hc1; sbox[8'hde]=8'h1d; sbox[8'hdf]=8'h9e;

sbox[8'he0]=8'he1; sbox[8'he1]=8'hf8; sbox[8'he2]=8'h98; sbox[8'he3]=8'h11;

sbox[8'he4]=8'h69; sbox[8'he5]=8'hd9; sbox[8'he6]=8'h8e; sbox[8'he7]=8'h94;

sbox[8'he8]=8'h9b; sbox[8'he9]=8'h1e; sbox[8'hea]=8'h87; sbox[8'heb]=8'he9;

sbox[8'hec]=8'hce; sbox[8'hed]=8'h55; sbox[8'hee]=8'h28; sbox[8'hef]=8'hdf;

sbox[8'hf0]=8'h8c; sbox[8'hf1]=8'ha1; sbox[8'hf2]=8'h89; sbox[8'hf3]=8'h0d;

sbox[8'hf4]=8'hbf; sbox[8'hf5]=8'he6; sbox[8'hf6]=8'h42; sbox[8'hf7]=8'h68;

sbox[8'hf8]=8'h41; sbox[8'hf9]=8'h99; sbox[8'hfa]=8'h2d; sbox[8'hfb]=8'h0f;

sbox[8'hfc]=8'hb0; sbox[8'hfd]=8'h54; sbox[8'hfe]=8'hbb; sbox[8'hff]=8'h16;

end

reg [31:0] rcon[0:9];

initial begin

rcon[0]=32'h01000000; rcon[1]=32'h02000000; rcon[2]=32'h04000000; rcon[3]=32'h08000000; rcon[4]=32'h10000000;

rcon[5]=32'h20000000; rcon[6]=32'h40000000; rcon[7]=32'h80000000; rcon[8]=32'h1b000000; rcon[9]=32'h36000000;

end

// xtime函数（标准有限域乘法）

function [7:0] xtime(input [7:0] x);

xtime = (x << 1) ^ ((x & 8'h80) ? 8'h1b : 8'h00);

endfunction

reg [3:0] round;

reg [2:0] step;

reg [127:0] state;

reg [127:0] rk;          // 优化点：只保存当前轮密钥，不保存全部11个

reg [127:0] sub_bytes_r; // S盒打拍寄存器

reg [31:0]  temp_word_r;

// ---------------------------------------------------------

// 核心优化：复用 S-box，全局只实例化 4 个，分 5 个周期读取！

// ---------------------------------------------------------

wire [31:0] sbox_in = (step == 0) ? state[31:0] :

(step == 1) ? state[63:32] :

(step == 2) ? state[95:64] :

(step == 3) ? state[127:96] :

{rk[23:16], rk[15:8], rk[7:0], rk[31:24]}; // Step 4 key expansion shift

wire [31:0] sbox_out;

assign sbox_out[7:0]   = sbox[sbox_in[7:0]];

assign sbox_out[15:8]  = sbox[sbox_in[15:8]];

assign sbox_out[23:16] = sbox[sbox_in[23:16]];

assign sbox_out[31:24] = sbox[sbox_in[31:24]];

// ---------------------------------------------------------

// 纯组合逻辑运算区

// ---------------------------------------------------------

// 1. 密钥扩展

wire [31:0] w0_c = rk[127:96] ^ temp_word_r ^ rcon[round-1];

wire [31:0] w1_c = w0_c ^ rk[95:64];

wire [31:0] w2_c = w1_c ^ rk[63:32];

wire [31:0] w3_c = w2_c ^ rk[31:0];

wire [127:0] next_rk_c = {w0_c, w1_c, w2_c, w3_c};

// 2. 行移位 (ShiftRows)

wire [127:0] shift_out_c;

assign shift_out_c[127:96] = {sub_bytes_r[127:120], sub_bytes_r[87:80], sub_bytes_r[47:40], sub_bytes_r[7:0]};

assign shift_out_c[95:64]  = {sub_bytes_r[95:88],  sub_bytes_r[55:48], sub_bytes_r[15:8], sub_bytes_r[103:96]};

assign shift_out_c[63:32]  = {sub_bytes_r[63:56],  sub_bytes_r[23:16], sub_bytes_r[111:104], sub_bytes_r[71:64]};

assign shift_out_c[31:0]   = {sub_bytes_r[31:24],  sub_bytes_r[119:112], sub_bytes_r[79:72], sub_bytes_r[39:32]};

// 3. 列混合 (MixColumns)

wire [127:0] mix_out_c;

assign mix_out_c[127:120] = xtime(shift_out_c[127:120]) ^ (xtime(shift_out_c[119:112])^shift_out_c[119:112]) ^ shift_out_c[111:104] ^ shift_out_c[103:96];

assign mix_out_c[119:112] = shift_out_c[127:120] ^ xtime(shift_out_c[119:112]) ^ (xtime(shift_out_c[111:104])^shift_out_c[111:104]) ^ shift_out_c[103:96];

assign mix_out_c[111:104] = shift_out_c[127:120] ^ shift_out_c[119:112] ^ xtime(shift_out_c[111:104]) ^ (xtime(shift_out_c[103:96])^shift_out_c[103:96]);

assign mix_out_c[103:96]  = (xtime(shift_out_c[127:120])^shift_out_c[127:120]) ^ shift_out_c[119:112] ^ shift_out_c[111:104] ^ xtime(shift_out_c[103:96]);

assign mix_out_c[95:88] = xtime(shift_out_c[95:88]) ^ (xtime(shift_out_c[87:80])^shift_out_c[87:80]) ^ shift_out_c[79:72] ^ shift_out_c[71:64];

assign mix_out_c[87:80] = shift_out_c[95:88] ^ xtime(shift_out_c[87:80]) ^ (xtime(shift_out_c[79:72])^shift_out_c[79:72]) ^ shift_out_c[71:64];

assign mix_out_c[79:72] = shift_out_c[95:88] ^ shift_out_c[87:80] ^ xtime(shift_out_c[79:72]) ^ (xtime(shift_out_c[71:64])^shift_out_c[71:64]);

assign mix_out_c[71:64] = (xtime(shift_out_c[95:88])^shift_out_c[95:88]) ^ shift_out_c[87:80] ^ shift_out_c[79:72] ^ xtime(shift_out_c[71:64]);

assign mix_out_c[63:56] = xtime(shift_out_c[63:56]) ^ (xtime(shift_out_c[55:48])^shift_out_c[55:48]) ^ shift_out_c[47:40] ^ shift_out_c[39:32];

assign mix_out_c[55:48] = shift_out_c[63:56] ^ xtime(shift_out_c[55:48]) ^ (xtime(shift_out_c[47:40])^shift_out_c[47:40]) ^ shift_out_c[39:32];

assign mix_out_c[47:40] = shift_out_c[63:56] ^ shift_out_c[55:48] ^ xtime(shift_out_c[47:40]) ^ (xtime(shift_out_c[39:32])^shift_out_c[39:32]);

assign mix_out_c[39:32] = (xtime(shift_out_c[63:56])^shift_out_c[63:56]) ^ shift_out_c[55:48] ^ shift_out_c[47:40] ^ xtime(shift_out_c[39:32]);

assign mix_out_c[31:24] = xtime(shift_out_c[31:24]) ^ (xtime(shift_out_c[23:16])^shift_out_c[23:16]) ^ shift_out_c[15:8] ^ shift_out_c[7:0];

assign mix_out_c[23:16] = shift_out_c[31:24] ^ xtime(shift_out_c[23:16]) ^ (xtime(shift_out_c[15:8])^shift_out_c[15:8]) ^ shift_out_c[7:0];

assign mix_out_c[15:8]  = shift_out_c[31:24] ^ shift_out_c[23:16] ^ xtime(shift_out_c[15:8]) ^ (xtime(shift_out_c[7:0])^shift_out_c[7:0]);

assign mix_out_c[7:0]   = (xtime(shift_out_c[31:24])^shift_out_c[31:24]) ^ shift_out_c[23:16] ^ shift_out_c[15:8] ^ xtime(shift_out_c[7:0]);

// 主状态机

always @(posedge sys_clk or negedge sys_rst_n) begin

if(!sys_rst_n) begin

round <= 4'd0;

step <= 3'd0;

state <= 128'd0;

rk <= 128'd0;

ciphertext <= 128'd0;

done <= 1'b0;

end else begin

if(start && round == 4'd0) begin

state <= plaintext ^ key;

rk <= key;

round <= 4'd1;

step <= 3'd0;

done <= 1'b0;

end else if(round >= 4'd1 && round <= 4'd10) begin

case(step)

3'd0: begin sub_bytes_r[31:0]   <= sbox_out; step <= 3'd1; end

3'd1: begin sub_bytes_r[63:32]  <= sbox_out; step <= 3'd2; end

3'd2: begin sub_bytes_r[95:64]  <= sbox_out; step <= 3'd3; end

3'd3: begin sub_bytes_r[127:96] <= sbox_out; step <= 3'd4; end

3'd4: begin temp_word_r         <= sbox_out; step <= 3'd5; end

3'd5: begin

rk <= next_rk_c;

state <= (round == 10 ? shift_out_c : mix_out_c) ^ next_rk_c;

round <= round + 1'b1;

step <= 3'd0;

end

endcase

end else if(round == 4'd11) begin

ciphertext <= state;

done <= 1'b1;

round <= 4'd0;

end

end

end

endmodule

// #########################################################################

// 子模块：串行化标准HMAC-SHA256

// #########################################################################

module hmac_sha256_standard(

input sys_clk,

input sys_rst_n,

input [127:0] msg,      // 输入消息（128位）

input [255:0] key,      // HMAC密钥（256位）

input start,

output reg [255:0] hmac,

output reg done

);

// HMAC状态机

localparam IDLE           = 4'd0;

localparam KEY_PAD        = 4'd1;

localparam IPAD_GEN       = 4'd2;

localparam INNER_BLOCK1   = 4'd3;

localparam INNER_BLOCK2   = 4'd4;

localparam INNER_DONE     = 4'd5;

localparam OPAD_GEN       = 4'd6;

localparam OUTER_BLOCK1   = 4'd7;

localparam OUTER_BLOCK2   = 4'd8;

localparam OUTER_DONE     = 4'd9;

localparam OUTPUT         = 4'd10;

reg [3:0] state;

reg [511:0] k_pad;

reg [511:0] k_ipad;

reg [511:0] k_opad;

reg [511:0] msg_block;

reg [255:0] inner_hash;

reg [255:0] inter_hash;

reg sha256_start;

reg sha256_init_enable;

wire [255:0] sha256_out;

wire sha256_done;

sha256_core u_sha256(

.sys_clk(sys_clk),

.sys_rst_n(sys_rst_n),

.msg_block(msg_block),

.start(sha256_start),

.init_hash(inter_hash),

.init_enable(sha256_init_enable),

.hash(sha256_out),

.done(sha256_done)

);

// HMAC主状态机

always @(posedge sys_clk or negedge sys_rst_n) begin

if(!sys_rst_n) begin

state <= IDLE;

k_pad <= 512'd0;

k_ipad <= 512'd0;

k_opad <= 512'd0;

msg_block <= 512'd0;

inner_hash <= 256'd0;

inter_hash <= 256'd0;

hmac <= 256'd0;

done <= 1'b0;

sha256_start <= 1'b0;

sha256_init_enable <= 1'b0;

end else begin

case(state)

IDLE: begin

done <= 1'b0;

if(start) state <= KEY_PAD;

end

KEY_PAD: begin

k_pad <= {key, 256'd0};  // 密钥左对齐补0

state <= IPAD_GEN;

end

IPAD_GEN: begin

k_ipad <= k_pad ^ {16{32'h36363636}};

state <= INNER_BLOCK1;

end

INNER_BLOCK1: begin

// 第一块：K_ipad（512位，无填充）

msg_block <= k_ipad;

sha256_start <= 1'b1;

sha256_init_enable <= 1'b0;  // 使用默认初始值

state <= INNER_BLOCK2;

end

INNER_BLOCK2: begin

sha256_start <= 1'b0;

if(sha256_done) begin

inter_hash <= sha256_out;

// 构造第二块：msg + 0x80 + 0... + 长度(128)

msg_block <= {msg, 1'b1, 319'd0, 64'd128};

sha256_start <= 1'b1;

sha256_init_enable <= 1'b1;  // 使用 inter_hash

state <= INNER_DONE;

end

end

INNER_DONE: begin

sha256_start <= 1'b0;

if(sha256_done) begin

inner_hash <= sha256_out;

state <= OPAD_GEN;

end

end

OPAD_GEN: begin

k_opad <= k_pad ^ {16{32'h5c5c5c5c}};

state <= OUTER_BLOCK1;

end

OUTER_BLOCK1: begin

// 第一块：K_opad（512位，无填充）

msg_block <= k_opad;

sha256_start <= 1'b1;

sha256_init_enable <= 1'b0;  // 使用默认初始值

state <= OUTER_BLOCK2;

end

OUTER_BLOCK2: begin

sha256_start <= 1'b0;

if(sha256_done) begin

inter_hash <= sha256_out;

// 构造第二块：inner_hash + 0x80 + 0... + 长度(256)

msg_block <= {inner_hash, 1'b1, 191'd0, 64'd256};

sha256_start <= 1'b1;

sha256_init_enable <= 1'b1;  // 使用 inter_hash

state <= OUTER_DONE;

end

end

OUTER_DONE: begin

sha256_start <= 1'b0;

if(sha256_done) begin

hmac <= sha256_out;

state <= OUTPUT;

end

end

OUTPUT: begin

done <= 1'b1;

state <= IDLE;

end

endcase

end

end

endmodule

// #########################################################################

// 子模块：面积优化版 SHA256 (滑动窗口架构，消除 1500+ 个寄存器)

// #########################################################################

module sha256_core(

input sys_clk,

input sys_rst_n,

input [511:0] msg_block,

input start,

input [255:0] init_hash,

input init_enable,

output reg [255:0] hash,

output reg done

);

function [31:0] Ch(input [31:0] x, y, z); Ch = (x & y) ^ (~x & z); endfunction

function [31:0] Maj(input [31:0] x, y, z); Maj = (x & y) ^ (x & z) ^ (y & z); endfunction

function [31:0] Sigma0(input [31:0] x); Sigma0 = {x[1:0],x[31:2]} ^ {x[12:0],x[31:13]} ^ {x[21:0],x[31:22]}; endfunction

function [31:0] Sigma1(input [31:0] x); Sigma1 = {x[5:0],x[31:6]} ^ {x[10:0],x[31:11]} ^ {x[24:0],x[31:25]}; endfunction

function [31:0] sigma0(input [31:0] x); sigma0 = {x[6:0],x[31:7]} ^ {x[17:0],x[31:18]} ^ (x >> 3); endfunction

function [31:0] sigma1(input [31:0] x); sigma1 = {x[16:0],x[31:17]} ^ {x[18:0],x[31:19]} ^ (x >> 10); endfunction

reg [31:0] SHA256_K[0:63];

initial begin

SHA256_K[0]=32'h428a2f98; SHA256_K[1]=32'h71374491; SHA256_K[2]=32'hb5c0fbcf; SHA256_K[3]=32'he9b5dba5;

SHA256_K[4]=32'h3956c25b; SHA256_K[5]=32'h59f111f1; SHA256_K[6]=32'h923f82a4; SHA256_K[7]=32'hab1c5ed5;

SHA256_K[8]=32'hd807aa98; SHA256_K[9]=32'h12835b01; SHA256_K[10]=32'h243185be; SHA256_K[11]=32'h550c7dc3;

SHA256_K[12]=32'h72be5d74; SHA256_K[13]=32'h80deb1fe; SHA256_K[14]=32'h9bdc06a7; SHA256_K[15]=32'hc19bf174;

SHA256_K[16]=32'he49b69c1; SHA256_K[17]=32'hefbe4786; SHA256_K[18]=32'h0fc19dc6; SHA256_K[19]=32'h240ca1cc;

SHA256_K[20]=32'h2de92c6f; SHA256_K[21]=32'h4a7484aa; SHA256_K[22]=32'h5cb0a9dc; SHA256_K[23]=32'h76f988da;

SHA256_K[24]=32'h983e5152; SHA256_K[25]=32'ha831c66d; SHA256_K[26]=32'hb00327c8; SHA256_K[27]=32'hbf597fc7;

SHA256_K[28]=32'hc6e00bf3; SHA256_K[29]=32'hd5a79147; SHA256_K[30]=32'h06ca6351; SHA256_K[31]=32'h14292967;

SHA256_K[32]=32'h27b70a85; SHA256_K[33]=32'h2e1b2138; SHA256_K[34]=32'h4d2c6dfc; SHA256_K[35]=32'h53380d13;

SHA256_K[36]=32'h650a7354; SHA256_K[37]=32'h766a0abb; SHA256_K[38]=32'h81c2c92e; SHA256_K[39]=32'h92722c85;

SHA256_K[40]=32'ha2bfe8a1; SHA256_K[41]=32'ha81a664b; SHA256_K[42]=32'hc24b8b70; SHA256_K[43]=32'hc76c51a3;

SHA256_K[44]=32'hd192e819; SHA256_K[45]=32'hd6990624; SHA256_K[46]=32'hf40e3585; SHA256_K[47]=32'h106aa070;

SHA256_K[48]=32'h19a4c116; SHA256_K[49]=32'h1e376c08; SHA256_K[50]=32'h2748774c; SHA256_K[51]=32'h34b0bcb5;

SHA256_K[52]=32'h391c0cb3; SHA256_K[53]=32'h4ed8aa4a; SHA256_K[54]=32'h5b9cca4f; SHA256_K[55]=32'h682e6ff3;

SHA256_K[56]=32'h748f82ee; SHA256_K[57]=32'h78a5636f; SHA256_K[58]=32'h84c87814; SHA256_K[59]=32'h8cc70208;

SHA256_K[60]=32'h90befffa; SHA256_K[61]=32'ha4506ceb; SHA256_K[62]=32'hbef9a3f7; SHA256_K[63]=32'hc67178f2;

end

reg [6:0] round;

reg [31:0] a,b,c,d,e,f,g,h;

// 优化点：使用 16 深度滑动窗口取代 64 深度大数组

reg [31:0] w[0:15];

reg [255:0] current_init;

// 滑动窗口的核心计算

wire [31:0] current_w = w[0]; // 当前轮恒定读取 w[0]

wire [31:0] new_w     = sigma1(w[14]) + w[9] + sigma0(w[1]) + w[0];

wire [31:0] temp1 = h + Sigma1(e) + Ch(e,f,g) + SHA256_K[round-1] + current_w;

wire [31:0] temp2 = Sigma0(a) + Maj(a,b,c);

always @(posedge sys_clk or negedge sys_rst_n) begin

if(!sys_rst_n) begin

round <= 7'd0;

hash <= 256'd0;

done <= 1'b0;

a<=0; b<=0; c<=0; d<=0; e<=0; f<=0; g<=0; h<=0;

current_init <= 256'd0;

end else begin

if(start && round == 7'd0) begin

if(init_enable) begin

current_init <= init_hash;

{a,b,c,d,e,f,g,h} <= init_hash;

end else begin

current_init <= 256'h6a09e667bb67ae853c6ef372a54ff53a510e527f9b05688c1f83d9ab5be0cd19;

{a,b,c,d,e,f,g,h} <= 256'h6a09e667bb67ae853c6ef372a54ff53a510e527f9b05688c1f83d9ab5be0cd19;

end

// 初始化前 16 个字

w[0]  <= msg_block[0*32 +:32]; w[1]  <= msg_block[1*32 +:32];

w[2]  <= msg_block[2*32 +:32]; w[3]  <= msg_block[3*32 +:32];

w[4]  <= msg_block[4*32 +:32]; w[5]  <= msg_block[5*32 +:32];

w[6]  <= msg_block[6*32 +:32]; w[7]  <= msg_block[7*32 +:32];

w[8]  <= msg_block[8*32 +:32]; w[9]  <= msg_block[9*32 +:32];

w[10] <= msg_block[10*32+:32]; w[11] <= msg_block[11*32+:32];

w[12] <= msg_block[12*32+:32]; w[13] <= msg_block[13*32+:32];

w[14] <= msg_block[14*32+:32]; w[15] <= msg_block[15*32+:32];

round <= 7'd1;

done <= 1'b0;

end else if(round >= 7'd1 && round <= 7'd64) begin

a <= temp1 + temp2;

b <= a;

c <= b;

d <= c;

e <= d + temp1;

f <= e;

g <= f;

h <= g;

// 滑动窗口移位

w[0] <= w[1];   w[1] <= w[2];   w[2]  <= w[3];  w[3]  <= w[4];

w[4] <= w[5];   w[5] <= w[6];   w[6]  <= w[7];  w[7]  <= w[8];

w[8] <= w[9];   w[9] <= w[10];  w[10] <= w[11]; w[11] <= w[12];

w[12]<= w[13];  w[13]<= w[14];  w[14] <= w[15]; w[15] <= new_w;

round <= round + 7'd1;

end else if(round == 7'd65) begin

hash <= {a + current_init[255:224], b + current_init[223:192],

c + current_init[191:160], d + current_init[159:128],

e + current_init[127:96],  f + current_init[95:64],

g + current_init[63:32],   h + current_init[31:0]};

done <= 1'b1;

round <= 7'd0;

end else if(round == 7'd0) begin

done <= 1'b0;

end

end

end

endmodule